AVN-AH8

AVN-AH8 8 Stereo Analogue Headphone Outputs Dante Interface, PoE

The AVN-AH8 multi-channel headphone amplifier is a 1U rack-mount which distributes 2 sets of stereo audio to up to 8 different sets of headphones, fed from the Dante AoIP network

The AVN-AH8 is a 1U rack-mount multi-channel headphone amplifier that distributes two sets of stereo audio to up to eight headphones, sourced from the Dante AoIP network. Each output features front-panel unbalanced 1/4” and 3.5 mm jack sockets with a rotary level control. The front-panel potentiometers adjust headphone volume from mute (fully anticlockwise) to 0 dB of gain (fully clockwise), allowing compensation for low or high Dante stream levels.

For ease of installation in enclosed racks, the rear panel also provides eight parallel 1/4” jacks.

Each of the eight professional headphone outputs can select between two stereo Dante sources, routed via Dante Controller. The level of each stereo stream/source can be adjusted ±12 dB using one of the two front-panel rotary controls. The unit supports AES67 operation and is Dante Domain Manager compliant.

The unit can be powered via PoE or a 12V DC input through a locking 2.5 mm DC connector with a minimum 2 A rating.

The front panel includes Power over Ethernet (PoE) and DC power LEDs, a network link status LED, and a clock LED. A recessed reset switch is also provided.

Typical Applications

A typical application is providing common headphone feeds directly from the AoIP network. Ideal for guests or musicians in a studio or recording environment.

Product Features

- 8 x front panel 1/4” & 3.5mm jack sockets.

- 8 x parallel connections on rear panel

- 8 x volume control.

- Switch selection for each headphone output between 2 dual-channel Dante sources.

- 2 x master level controls, one for each stereo Dante input.

- 1 x RJ45 Dante connector (100 Mb/s Ethernet Port).

- PoE, Link, DC and Clock LED status indicators.

- Configuration using Dante Controller.

- AES67 operation & Dante Domain Manager compliant.

- Powered by PoE.

- 1U 19" rack-mount form factor.

- Aluminium construction

TECHNICAL SPECIFICATIONS

Network and AoIP

- AoIP Standard:		Dante

- Number of Channels:	4 Receive

- Number of Streams:	2 Receive

- Sample Rates:		44.1kHz, 48 kHz, 88.2kHz, 96kHz

- Encoding Fornat:	Linear PCM, 16, 24 or 32 bit

- AES67 Support:		Yes

- Connectivity:		1 x RJ45

- Speed:			100mps

PoE Power

- Standard:		802.3af

- Class:			0

- PD Power Range:	0.44 W to 12.94 W

- Typical PSE Power Usage:  6 W

Front Panel Connections & Controls

- LED Indicators	Clock LED – indicates valid clock on AoIP network PoE LED – indicates power presence Link LED – indicates network link status DC LED – indicates DC power applied

- Reset:					Recessed system reset button

- Headphone Outputs:			8 x dual 1/4”/3.5mm jack sockets

- Headphone Output Volume Controls:	8 x dual log potentiometers

- Headphone Input Source Selection:	8 x toggle switches for individual source selection between two stereo inputs

Rear Panel Connections

- Parameter			Description

- Network			Ethernet port (100Mbps capable) for data and PoE connection

- Headphone Parallel Outputs:	8 x ¼” jack sockets for parallel headphone outputs 1-8

Dante Network to Headphone Input Specifications

Master Level Adjustment	±12dB

Headphone Output Specifications

- Parameter			Description

- Output Impedance:		Capable of driving 150mW into 32Ω to 600Ω headphones

- Maximum Output Level:	+18dBu unclipped

- D to A Line-Up:	0dBFS = 	+18dBu (Volume = 100%, Master level = 50% = 0dB)

- Volume Pot Range		-80dB (Volume = 0%) to 0dB (Volume =100%)

- Frequency Response:		20Hz to 20kHz, +0/-0.5dB (ref 1kHz)

- THD+N:				<0.07%, -10dBFS input, volume = 100%, master level = 50% = 0dB, 20Hz to 20kHz, 20kHz BW

- Dynamic Range:			108dB, volume = 100%, master level = 50% = 0dB, 20kHz BW

Headphone Jack Output Pin-out

- Pin	Function

- Tip	Left

- Ring	Right

- Screen	0V Common

Equipment Type

AVN-AH8 8 Stereo Analogue Headphone Outputs Dante Interface, PoE

Physical Specification

- Dimensions (Raw):	48cm (W) x 11cm (D) x 4.3cm (H)(1U) 19" (W) x 4.3" (D) x 1.7” (H)

- Dimensions (Boxed):	58.5cm (W) x 22.5cm (D) x 7cm (H) 23” (W) x 8.9” (D) x 2.8” (H)

- Weight:	Nett: 1.4kg Gross: 2.0kg Nett: 3.1lbs Gross: 4.4lbs