AVN-M8R

8 Mic/Line Input Dante Interface, Dual PoE

The AVN-M8R multi-channel Dante interface distributes 8 balanced mic/line inputs to 8 channels of Dante/AES67 on the AoIP network.

The AVN-M8R multi-channel Dante interface is a 19” 1U rack-mount unit which distributes 8 balanced mic/line inputs to 8 channels of Dante/AES67 on the AoIP network. Each mic/line input can be controlled remotely via a web server.

The front panel includes clear LED indicators for power (PoE), network status and clock synchronisation, along with dedicated LEDs for secondary port link and PoE monitoring.

On the rear 8 XLR balanced Neutrik connectors provide the professional mic/line interface with phantom power indicators. The unit is powered via PoE for installation savings, reliability and efficiency. Primary and secondary 1Gb RJ45 ports are offered as standard to provide full power and data redundancy, ensuring an uninterrupted reliable solution in critical environments.

The GPIO and VGPIO can be used to trigger events over the network between devices, such a microphone muting and other events.

Fully compatible with the Dante AoIP standard, the AVN-M8R allows for flexible audio routing via Dante Controller. Additionally, Ember+ can be used as a control protocol for real-time status updates and remote control. The unit supports AES67 operation and is Dante Domain Manager compliant offering a simple and intuitive user experience.

A built-in web server enables easy configuration of each input type on the AVN-M8R. This provides intuitive control, presenting all 8 mic/line inputs as individual channel strips, each showing its corresponding microphone or line settings. Selecting a channel strip allows you to edit and configure its parameters. A convenient “Copy Settings” function lets you duplicate settings across your choice of channels. Each channel strip shows the hardware channel number, the “Dante Input Name,” and a user-defined “Local Input Name,” making it easy to identify channels regardless of the naming conventions used. For more information on the web server features and adjustable mic/line parameters, see section below.

With its robust feature set, flexible connectivity, and remote management capabilities, the AVN-M8R is the perfect solution for broadcasters, recording studios and live sound applications that require high-quality professional AoIP integration from their mic or line sources.

Typical Applications

A typical application would be in a live conference, meeting, or concert, where multiple microphones and line-level sources are deployed around the venue to capture speakers, performers, or ambient sound. Its remote-control capability allows easy adjustment of gain and phantom power, even when the unit is discreetly hidden.

Product Features

- 8 x Mic/Line Inputs Using Neutrik Connectors.

- Phantom Power with Indicators for Each Mic Input.

- Remote Individual Input Control; such as, Mic Gain and Line-up Control Via the Web Server.

- 2 x RJ45 Connector (1 Gb/s Ethernet Port) for PoE Power and Network Redundancy.

- PoE, Link and Clock LED Status Indicators.

- Configuration Using Dante Controller & Dante Domain Manager Compliant.

- AES67 Compatible.

- 1U 19" Rack-Mount Form Factor.

- Aluminium Construction.

Web Server Features

- Real-Time System Information and Status Monitoring.

- Firmware Update Process.

- Network Configuration.

- User Defined Device Identification.

- Password Protection.

- GPIO/VGPIO Routing.

- SNMP Configuration.

- Save and Load Configuration for Quick Deployment.

- Export System Logs for Diagnostics and Support.

Adjustable Mic/Line Parameters

- Mic Phantom Power.

- Preamp Mic Gain (16dB to 76dB) in 3db Increments.

- 5 Levels of Input Line-up (+15dBu to +24dBu = 0dBFS).

- Mic or Line High Pass Filter.

- Mic or Line High Pass Corner Frequency.

- Mic or Line Signal Presence Threshold (dB).

- Mic or Line Signal Presence Timeout.

- Global Status LED's Enable/Disable (Dark Mode).

Product Category

Microphone Amplifiers

TECHNICAL SPECIFICATIONS

Network and AoIP

- AoIP Standard: Dante

- Number of Channels: 8 Transmit

- Number of Streams: 8 Transmit

- Sample Rates: 44.1kHz, 48kHz, 88.2kHz, 96kHz

- Encoding Format: Linear PCM, 16, 24 or 32-bit

- AES67 Support: Yes

- Connectivity: 2 × RJ45

- Speed: 1Gbps or 100Mbps

- Network Modes: Switched or Redundant

- Dante Domain Manager Ready: Yes

Mic/Line Inputs

- Input Impedance (Mic Mode): 2.5kΩ Electronically Balanced

- Input Impedance (Line Mode): >10kΩ Electronically Balanced

- Preamp Gain (Mic Mode): User selectable from 16dB to 76dB in 3dB steps (ref. +18dBu = 0dBFS)

- 0dBFS Line-Up (Mic Mode): -58dBu @ maximum preamp gain, +2dBu @ minimum preamp gain

- 0dBFS Line-Up (Line Mode): User selectable +15dBu / +18dBu / +20dBu / +22dBu / +24dBu = 0dBFS

- Frequency Response (Mic Mode): +0/-0.2dB, 20Hz to 42kHz (ref. 40dB gain @ 1kHz)

- Frequency Response (Line Mode): +0/-0.2dB, 20Hz to 42kHz (ref. 0dBu @ 1kHz)

- THD+N (Mic Mode): <-90dBFS, -52dBu, 20Hz to 20kHz, 40dB gain, 20kHz BW

- THD+N (Line Mode): <-100dBFS, -6dBu, 20Hz to 20kHz, 20kHz BW (ref. +24dBu = 0dBFS)

- Equivalent Input Noise (Mic Mode): <-143dBFS (-125dBu), ref. 76dB preamp gain, Rs = 200Ω

- Noise (Line Mode): <-100dBFS, 20kHz BW, Rs = 200Ω

- Common Mode Rejection (CMRR): >60dB @ 1kHz

- Phantom Power (Mic Mode): +48V

- High Pass Filter: 12dB/octave, user-selectable frequency

Balanced Line Input XLR Pin-out

- Pin 1: Chassis Ground / Screen

- Pin 2: Input Phase / Positive

- Pin 3: Input Non-Phase / Negative

PoE Power

- Standard: IEEE 802.3af

- Class: 0

- PD Power Range: 0.44W to 12.94W

- Typical PSE Power Usage: 10W

- Maximum PSE Power Usage: 15.4W

Physical Specification

- Dimensions (Unboxed): 48.0cm (W) × 11.0cm (D) × 4.3cm (H) (1U)

- Dimensions (Unboxed): 19.0" (W) × 4.3" (D) × 1.7" (H)

- Dimensions (Boxed): 59.5cm (W) × 22.5cm (D) × 7.0cm (H)

- Dimensions (Boxed): 23.4" (W) × 8.9" (D) × 1.7" (H)

- Weight (Nett): 1.02kg (2.24lbs)

- Weight (Gross): 1.41kg (3.11lbs)

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/avn-m8r/