AVN-DIO03

Dante to Headphone Outputs (1/4” & 3.5mm Jacks) with Volume Control & Limiter

Dante AoIP network-to-headphones convertor. It features a single stereo headphone output available on two jacks (1/4″ and 3.5mm) with volume control.

The AVN-DIO03 is a Dante AoIP network-to-headphones convertor from the Sonifex DIO range. The unit features a single stereo headphone output, accessible via both 1/4” and 3.5 mm jacks, with a front-panel potentiometer to adjust volume from mute (fully anticlockwise) to +6 dB gain (fully clockwise), useful for boosting low Dante stream levels.

A built-in limiter with adjustable threshold and LED indication protects hearing by preventing over-levels. The limit can be set between approximately -12dBu and +6dBu using a small adjustment tool. When limiting is active, the blue limit LED illuminates, prompting the user to reduce headphone volume.

Audio and power (PoE) are delivered via a Neutrik EtherCON® connector, allowing direct connection to a Dante network. The unit incorporates Audinate Dante chipsets and is compliant with AES67.

Like all Sonifex DIO interfaces, it provides a simple plug-and-play solution for connecting analogue and digital audio equipment to Dante networks, with configuration handled via Dante Controller. It is housed in a rugged aluminium enclosure with provisions for desk mounting, and an optional accessory, AVN-DIORK, is available to install multiple units on a 19″ rack-mount shelf.

Note: The headphone output of this device is unbalanced, resulting in 6 dB lower performance compared with balanced AVN-DIO products.

Typical Applications

Provides a stereo, or two mono, headphone feeds directly from the Dante AoIP network. Ideal for musicians in a studio or recording environment, or for engineers requiring physical monitoring of two Dante channels.

Product Features

- 1/4-inch and 3.5mm jack analogue headphone outputs.

- Headphone volume control.

- Limiter on/off, threshold control and LED indicator.

- Neutrik EtherCon® Ethernet connection.

- Fully Dante compliant.

- AES67 compatible.

- Wide dynamic range D/A conversion.

- Powered via PoE (Power over Ethernet).

- Rack mount option with the AVN-DIORK.

- Aluminium construction.

Product Category

Headphone Amplifiers

TECHNICAL SPECIFICATIONS

Headphone Jack Output Pin-out

- Tip: Left

- Ring: Right

- Screen: 0 V common

Headphone Output – Jack

- Output Impedance: Capable of driving 150 mW into 32 Ω to 600 Ω headphones

- Maximum Output Level: +16 dBu

- D to A Line-Up: -6 dBFS = +16 dBu (max volume)

- Gain: Mute (min. volume) to +6 dB (max volume)

- Frequency Response: 20 Hz to 20 kHz, +0/-0.5 dB (Ref. 1 kHz)

- THD+N: <0.005%, -14 dBFS input, max volume, 20 Hz to 20 kHz, 20 kHz BW

- Dynamic Range: 114 dBu, 20 kHz BW, 0 dB gain

- Crosstalk: <100 dB

- Audio Limiter Range: -12 dBu to +6 dBu

Dante

- Sample Rates: 44.1 kHz, 48 kHz, 88.2 kHz and 96 kHz

- Encoding: PCM 16, PCM 24 and PCM 32

PoE Power

- Standard: 802.3af

- Class: 0

- PD Power Range: 0.44 W to 12.94 W

- Typical PSE Power Usage: 4 W

- Max PSE Power Usage: 15.4 W

Physical Specification

- Dimensions (Raw): 10.6 cm (W) × 7.3 cm (D) × 4.3 cm (H)

- Dimensions (Raw): 4.2" (W) × 2.9" (D) × 1.7" (H)

- Dimensions (Boxed): 17.4 cm (W) × 9.5 cm (D) × 5.6 cm (H)

- Dimensions (Boxed): 6.9" (W) × 3.7" (D) × 2.2" (H)

- Weight: Nett: 0.2 kg. Gross: 0.3 kg

- Weight: Nett: 0.44 lbs. Gross: 0.66 lbs

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/avn-dio03/