AVN-CU4-RAV

Configurable RAVENNA/AES67; Commentator Unit, 4 Commentators

The AVN-CU4-RAV is a portable, configurable commentator unit. Using RAVENNA/AES67 AoIP together with 4 wire connections the AVN-CU4-RAV bridges the gap between old and new technology.

The AVN-CU4-RAV was designed from the ground up to be totally flexible in operation. The use of RAVENNA/AES67 AoIP means that inputs and outputs can be added as required (up to a max of 16 per unit). Any physical input can be combined, mixed, and routed to any output, giving you complete flexibility to create fully customisable cross-points. Additionally, you can choose which of those cross-points need to be controlled by the use of rotary encoders.

Using a built-in web GUI, up to 48 encoders (8 nameable pages of 6 rotary encoders) can be assigned to inputs, outputs or cross-points. Encoders can control the volume and pan of headphones and volume of the assigned inputs, outputs or cross-points. Each rotary encoder has a separate colour-coded meter section showing the channel name, detailed level metering and left/right panning on a bright daylight reading display. The colours are configurable to quickly identify particular source groups, so headphone source selection becomes intuitive.

The unit can be fully remotely controlled from the web interface, which mimics the front panel of the unit and includes live metering. All the controls can be operated as if at the unit and there are front-panel lock-out options for every button and encoder, with ‘quick lock-out’ for the rows of buttons and pages of encoders.

The AVN-CU4-RAV has 4 x locking mic/line inputs with +48V phantom power indication and 4 x headphone outputs on locking 6.35mm jack sockets, suitable for operation by four commentators.

The unit supports full redundancy, using Power over Ethernet+ (PoE+), mains AC or 4 pin XLR 12V DC.

Metering is available per input/output, with output metering configurable as, pre, or post level adjustment. An adjustable limiter is available on every output and is applied automatically to prevent signal clipping.

The unit provides multiple 4-wire connections for integration with external systems:

- 4 × analogue line inputs (locking XLR)

- 6 × analogue line outputs (XLR)

- 1 × RJ45 AES3 stereo input

- 1 × RJ45 AES3 stereo output

All 4-wire connections can be freely assigned as programme, talkback, mic (line level), PA, or zone signals. They can also be used as insert or breakout points to and from the AoIP network.

The unit has dual redundant network ports on both RJ45 (Neutrik EtherCON® connectors) and SFP cages for long fibre runs. The AVN-CU4 support three network modes: Switched, Redundant, and Remote mode. In Switched mode, all ports carry both audio and control traffic on a single VLAN. Redundant mode separates audio across primary and secondary VLANs for resilience, but keeps control traffic only on the primary network. Remote mode builds on this by fully separating audio and control, introducing a dedicated “Control VLAN” while maintaining redundant Dante audio on the primary and secondary Ethernet ports. In this configuration, audio remains on the Ethernet ports, while the SFP ports are used exclusively for control, improving traffic isolation and making network design more flexible.

A total of 10 configurable GPIO connections are available via a 15-way D-type connector, including one switched changeover output. Both physical GPIO and virtual GPIO (VGPIO) can be configured within a matrix to trigger system actions, including muting and other automated functions. VGPIO also allows control between network-connected AVN-CU units.

The complete configuration of the unit, including all encoder settings, button setups and Dante mix matrix audio routes, can be saved off to a text editable .json file. Up to 8 of these files can be stored on the unit and loaded from the main Menu. This enables the unit to be pre-configured for any venue, or event and with these setups instantly recalled onsite.

The Front Panel Consists Of:

- 4 x On-Air buttons can be used to connect mic audio to the main output, either over AoIP or via the AES digital audio connection.

- 2 x Menu buttons can be used to access limited setup options on the TFT display.

- 4 x Page buttons change the display and encoders to monitor an additional set of sources, cross-points or outputs. Up to 8 pages can be pre-programmed, e.g. one for talkback inputs, one for outputs, one to monitor other sources.

- 4 x Cough buttons take the commentator off-air while pressed.

- 2 x User buttons can be programmed to perform any function.

- 8 x T/B (talkback) buttons can be configured to initiate talkback over AoIP or AES digital audio connection, using 4 x talkback buses. The talkback buttons operate with lazy talkback, taking the commentator off-air when invoked.

- 12 x rotary encoders.

The illuminated ‘Sonifex’ logo acts as a power indication and there are illuminated LEDs to indicate network clock, AoIP primary and secondary link, PoE primary and secondary status and when DC power active.

Typical Applications

Used in sports grounds and arenas by commentators to produce commentary & hear talkback, 16 x 16 matrix mixer with configurable controls for live events.

Product Features

- Web GUI for configuration & full remote control.

- Lock-out of all individual controls and/or button row/encoder pages.

- PoE+ using 2 x Neutrik EtherCON® RJ45s.

- Dual redundant network ports on RJ45 and SFP.

- Remote modes for flexible audio and control network separation on SFP.

- 4 x locking mic/line inputs with +48V phantom power indication.

- 4 x headphone outputs on locking 6.35mm jack sockets.

- 4 x analogue inputs and 6 analogue outputs on XLR.

- 1 x stereo AES/EBU input and output on RJ45s.

- 4 x talkback buses.

- On-air buttons with lazy talkback ability.

- Supports up to 16 x input and output streams and 16 input and output multicast flows/streams.

- Configurable headphone source selection.

- 12 x rotary encoders to control volume, or signal panning, and colour coded with the screen.

- 12 x detailed colour displays showing channel name, level metering & limit indication.

- Fully configurable buttons and rotary encoders, for inputs and outputs.

- 4 x pages of encoder settings, e.g. one for talkback inputs, one for outputs, one to monitor other sources.

- Metering per input/output, with output configured as pre or post level adjustment.

- Built-in line-up tone generator.

- Limiter available on every output.

- Output metering section shows limiter indication, the name of the output & level metering.

- 10 x configurable GPIO with 1 x switched changeover output.

- GPIO & VGPIO matrix programmable with events.

Product Category

RAVENNA Commentary

TECHNICAL SPECIFICATIONS

Network and AoIP

- AoIP Standard: RAVENNA/AES67

- Number of Channels: 16 receive, 16 transmit

- Number of Streams: 16 receive, 16 transmit

- Sample Rate: 48 kHz

- Format: Linear PCM, 16 or 24 bit

- AES67 Support: Yes

- Connectivity: 2 × RJ45 and 2 × SFP

- Speed: 1 Gbps and 100 Mbps

- Network Modes: Switched or redundant

- Network Manager: ANEMAN (Merging Technologies)

Mic/Line Inputs

- Input Impedance (Mic Mode): 2.5 kΩ electronically balanced

- Input Impedance (Line Mode): >10 kΩ electronically balanced

- Preamp Gain (Mic Mode): User selectable 0 dB to 60 dB in 3 dB steps

- 0 dBFS Line-up (Mic Mode): -58 dBu @ max preamp gain, +2 dBu @ min preamp gain

- 0 dBFS Line-up (Line Mode): User selectable +15 dBu / +18 dBu / +20 dBu / +22 dBu / +24 dBu = 0 dBFS

- Frequency Response (Mic Mode): +0/-0.2 dB, 20 Hz to 22 kHz, Ref. 40 dB gain @ 1 kHz

- Frequency Response (Line Mode): +0/-0.2 dB, 20 Hz to 22 kHz, Ref. 0 dBu @ 1 kHz

- THD+N (Mic Mode): <-90 dBFS, -64 dBFS, 20 Hz–20 kHz, 40 dB gain, 20 kHz BW

- THD+N (Line Mode): <-98 dBFS, -30 dBFS, 20 Hz–20 kHz, 20 kHz BW

- Equivalent Input Noise (Mic Mode): 127 dB, Ref. 60 dB preamp gain, Rs = 200 Ω

- Noise (Line Mode): <-110 dBFS, 20 kHz BW, Rs = 200 Ω

- C.M.R.R.: >60 dB

Stereo Digital Input

- Input Impedance: 110 Ω

- Supported Input Rates: 32 kHz, 44.1 kHz, 48 kHz, 88.2 kHz, 96 kHz, 176.4 kHz and 192 kHz

Stereo Digital Output

- Output Impedance: 110 Ω

- Supported Output Rates: 48 kHz

Analogue Line Inputs

- Input Impedance: >20 kΩ electronically balanced

- 0 dBFS Line-up: User selectable +15 dBu / +18 dBu / +20 dBu / +22 dBu / +24 dBu = 0 dBFS

- Frequency Response: +0/-0.2 dB, 20 Hz to 22 kHz, Ref. 0 dBu @ 1 kHz

- THD+N: <-110 dBFS, -30 dBFS, 20 Hz–20 kHz, 20 kHz BW

- Noise: <-110 dBFS, 20 kHz BW, Rs = 200 Ω

- C.M.R.R.: >60 dB

Analogue Line Outputs

- Output Impedance: <50 Ω

- 0 dBFS Line-up: User selectable 0 dBFS = +15 dBu / +18 dBu / +20 dBu / +22 dBu / +24 dBu

- Frequency Response: +0/-0.5 dB, 20 Hz to 22 kHz, Ref. 0 dBu @ 1 kHz

- THD+N: <-110 dBFS, -30 dBFS, 20 Hz–20 kHz, 20 kHz BW

- Noise: <-110 dBFS, 20 kHz BW

Headphone Outputs

- Drive Capability: Drives 150 mW into 32 Ω to 600 Ω headphones

- 0 dBFS Line-up: Fixed 0 dBFS = +18 dBu

- Frequency Response: +0/-0.2 dB, 20 Hz to 22 kHz, Ref. 0 dBu @ 1 kHz

- THD+N: <-108 dBFS, -30 dBFS, 20 Hz–20 kHz, 20 kHz BW

- Noise: <-110 dBFS, 20 kHz BW

Main Panel Operational Controls & Indicators

- Power LED: Sonifex logo illuminates when power is present

- Pushbuttons: 24 × illuminated buttons that can be assigned to any of the following functions and configured: On-Air, Talkback, Page #, Page Cycle, GPO, Cough, Menu and Brightness

- Display: 2 × 480 × 107 pixel colour TFT displays showing pan status, level, metering, control type, source/mix/destination names and main output metering

- Status LEDs: Clock, AoIP status, PoE status and PSU status LEDs to monitor power and connectivity. Can be disabled if required

- Rotary Encoders: 12 × robust soft-touch rotary encoders with adjacent configurable light bars for group/source/commentator grouping and identification

Front Panel Connections

- Headphone Outputs: 4 × Neutrik ¼" (6.35 mm) locking jack sockets

- Microphone/Line Inputs: 4 × Neutrik 3-pin latching XLR sockets

Rear Panel Connections

- Stereo Digital Input: RJ45 socket (electronically balanced)

- Stereo Digital Output: RJ45 socket (electronically balanced)

- Analogue Line Inputs: 4 × Neutrik locking XLR sockets

- Analogue Line Outputs: 6 × Neutrik locking XLR plugs

- GPIO Port: 15-way 'D'-type socket

- Network: 2 × 1 Gbps Neutrik EtherCON receptacles with PoE, maximum 12 W; 2 × SFP slots for alternative interfaces

- DC Input: Neutrik 4-pin XLR plug, 12 VDC, maximum 12 W

- Mains Input: Filtered IEC, 85–264 VAC, 47–63 Hz, 60 W max

- Fuse Rating: Anti-surge fuse, 2 A, 20 × 5 mm

PoE Power

- Standard: 802.3at Type 2

- Class: 4

- PD Power Range: 12.95 W to 25.50 W

- Typical PSE Power Usage: 20 W

- Max PSE Power Usage: 30 W

Physical Specification

- Dimensions (Raw): 25.7 cm (W) × 22.9 cm (D) × 9.2 cm (H, max), 6.5 cm (H, front), 8.9 cm (H, rear)

- Dimensions (Boxed): 37.7 cm (W) × 34 cm (D) × 17.5 cm (H)

- Weights: Nett: 2.9 kg. Gross: 3.6 kg

Accessories

- AVN-DCX60: DC power supply for AVN range, 4-pin XLR socket, 60 W

- AVN-TC1 (Case Only): Transport case with AVN-CU4 and power supply cut-out

- CM-GM2: Professional gooseneck condenser microphone

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/avn-cu4-rav/