AVN-DIO010-12G

Dante® to 12G/6G/3G/HD/SD-SDI Embedder/De-Embedder

The AVN-DIO10-12G can be used for simultaneous embedding and de-embedding. This simple plug and play audio/video interface provides a convenient and elegant method of connecting legacy 12G/6G/3G/HD/SD-SDI equipment to the Dante® AoIP audio network.

An upgrade to the popular AVN-DIO10, the AVN-DIO10-12G adds support for the full SDI range, from SD-SDI up to 12G-SDI. It performs simultaneous embedding and de-embedding of up to 64 audio channels, providing a simple plug-and-play audio/video interface for connecting 12G/6G/3G/HD/SD-SDI equipment to a Dante® AoIP audio network.

The unit de-embeds up to 64 audio channels from an SDI feed and places them on Dante channels 1–64 (configured via Dante Controller). At the same time, it takes the 64 Dante input channels mapped to the device and re-embeds them onto the SDI output.

Two operating modes are available: Loop-through mode, where the SDI input is passed directly to the output, and Generator mode, where input and output are independent and the SDI output is supplied by an internal SDI video generator. Generator mode allows the unit to act as an embedder without an SDI feed, or as separate de-embedder and embedder with different input/output formats. All settings are configurable through the web UI.

The unit supports embedded audio at 48 kHz and 96 kHz, allowing different sample rates on the de-embed and embed paths. At 96 kHz, SDI channel capacity is halved, with each audio group carrying two channels rather than four at 48 kHz.

The web UI allows Dante channels to be embedded onto the SDI output on a per-channel-pair basis and offers two options: Insert Mode enabled, where embedding selectively overwrites existing SDI audio per channel pair, and Insert Mode disabled, where incoming SDI audio channels are cleared before selective embedding is applied.

Test Tone mode outputs 1 kHz, 2 kHz, 3 kHz and 4 kHz tones on channels 1–4 respectively for any group with embedding enabled. A separate de-embedder test mode places a 1 kHz tone on the transmitting Dante® channels, enabling downstream SDI audio I/O testing without Dante® or SDI sources.

Power is provided via Power over Ethernet (PoE) using Neutrik® EtherCON connectors, with primary and secondary ports for power and data redundancy. Front panel LEDs indicate network clock status, AoIP link activity, and PoE power presence for clear operational monitoring. The unit incorporates Audinate Dante chipsets and is compliant with AES67 and Dante Domain Manager.

Like all Sonifex DIO interfaces, it provides a simple plug-and-play solution for connecting analogue and digital audio equipment to Dante networks, with configuration handled via Dante Controller. It is housed in a rugged aluminium enclosure with provisions for desk mounting, and an optional accessory, AVN-DIORK, allows multiple units to be installed on a 19″ rack-mount shelf.

Typical Applications

Taking audio embedded on an SDI camera feed, or any legacy SDI feed, and de-embedding it to the Dante network, adding Dante audio feeds to an SDI signal for distribution around a facility. Houses of worship - embed multi-language services for distribution. De-embed audio from live HD football feeds to provide multi-language radio commentaries, de-embed audio to pass to a Dante based speaker system. Embed remixed 5.1 + stereo music onto HD concert footage.

Product Features

- Dante AoIP

- 1 x 12G/6G/3G/HD/SD-SDI input.

- 1 x Re-clocked 12G/6G/3G/HD/SD-SDI output.

- Dual redundant primary and secondary Dante network ports using Neutrik EtherCon® ethernet connectors.

- Powered via PoE (Power over Ethernet) with PoE dual redundancy.

- Fully Dante compliant device.

- AES67 compatible.

- Dante Domain Manager compliant.

- Web interface for configuration.

- Clock, SDI Lock, PoE and Sync LEDs.

- Overwrite or insert into existing SDI audio groups.

- Test tones available on embedded outputs.

- SDI audio sample rate support at 48kHz.

- All available Dante sample rates supported.

- Sample rate conversion of audio between Dante and SDI.

- Dante clock domain can be optionally synchronised from the SDI clock.

- 1U rack-mount option with the AVN-DIORK.

- Aluminium construction.

Product Category

Dante Embedding & De-Embedding

TECHNICAL SPECIFICATIONS

Parameter SDI Input

- Input Impedance: 75 Ω unbalanced.

- SDI Supported Standards: 270 Mbps SMPTE-259M-C (SD); 1.485 or 1.4835 Gbps SMPTE-292M (HD); 2.97 or 2.967 Gbps SMPTE-424M (3G); 5.94 or 5.934 Gbps SMPTE-2081 (6G); 11.88 or 11.868 Gbps SMPTE-2082 (12G)

- Supported Video Formats: 525/59.94 (SMPTE-125M); 625/50 (ITU-R BT.656); 720p/23.98, 24, 25, 29.97, 30, 50, 59.94, 60 (SMPTE-296M); 1035i/59.94, 60 (SMPTE-260M); 1080i/50, 59.94, 60 (SMPTE-274M); 1080p/23.98, 24, 25, 50, 59.94, 60 (SMPTE-274M); 1080pSF/23.98, 24, 25, 29.97, 30 (RP-211); 1080i/50 (SMPTE-295M); 1080p/50 (SMPTE-295M); 2160p/23.98, 24, 25, 29.97, 30, 50, 59.94, 60 (SMPTE-2036)

- Embedded Audio: 48 kHz, synchronous; SMPTE-272M-ABC, SMPTE-299M and SMPTE-299-2 (de-embedding only)

- Supported Image Mapping: SMPTE-425M-AB

Parameter SDI Output

- Output Impedance: 75 Ω unbalanced

- Alignment Jitter: <0.2 UI

- Output Level: 800 mV ±10%

- Return Loss: TBC

- SDI Supported Standards: Loop-through – output follows input; Generator – see supported standards above

- Supported Video Formats: Loop-through – output follows input; Generator – see supported formats above

- Embedded Audio: 48 kHz, synchronous; SMPTE-272M-C and SMPTE-299M

Network and AoIP

- AoIP Standard: Dante

- Channels: 64 receive, 64 transmit @ 44.1/48 kHz; 32 receive, 32 transmit @ 88.2/96 kHz; 16 receive, 16 transmit @ 176.4/192 kHz

- Flows: 32 receive, 32 transmit

- Sample Rates: 44.1 kHz, 48 kHz, 88.2 kHz, 96 kHz, 176.4 kHz and 192 kHz

- Encoding: PCM 16, PCM 24 and PCM 32

- AES67 Support: Yes

- Connectivity: 2 × etherCON (RJ45 compatible)

- Speed: 1 Gbps or 100 Mbps

- Network Modes: Switched or redundant

- Dante Domain Manager Ready: Yes

- Clock Source: Internal (PTP Leader), network PTP Leader or from SDI input (Sync to External)

PoE Power

- Standard: 802.3af

- Redundancy: Yes

- Class: 0

- PD Power Range: 0.44 W to 12.94 W

- Typical PSE Power Usage: 6 W

- Max PSE Usage: 15.4 W

Physical Specification

- Dimensions (Raw): 14.0 cm (W) × 13.6 cm (D) × 4.2 cm (H); 5.5" (W) × 5.4" (D) × 1.7" (H)

- Dimensions (Boxed): 17.8 cm (W) × 17.0 cm (D) × 5.6 cm (H); 7.0" (W) × 6.7" (D) × 2.2" (H)

- Weight: Nett: 0.36 kg. Gross: 0.55 kg. Nett: 0.79 lbs. Gross: 1.21 lbs

Accessories

- AVN-DIORK: AVN-DIO 1U 19" rack kit (5 × small DIO or 3 × large DIO)

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/avn-dio10-12g/