AVN-DIO09

Microphone Input to Dante®

Microphone input to Dante audio converter. It features a single, full-featured, high-quality mic preamp, plus a Neutrik EtherCon® connector for direct connection to a Dante AoIP network.

The AVN-DIO09 is an ultra-high quality single microphone to Dante® interface from the Sonifex DIO range, featuring a balanced analogue XLR input with a high-performance mic preamp for seamless integration with Dante Audio-over-IP networks. It delivers A/D conversion with a world-class equivalent input noise (EIN) of 129dB.

The microphone input includes coarse and fine gain adjustment, a switchable high-pass filter, phantom power, and a tri-colour level LED for accurate signal monitoring. Coarse gain is selectable between 14dB and 44dB via a front panel switch, while fine gain provides an additional 0dB to 42dB of adjustment.

The high-pass filter can be enabled to attenuate frequencies below 125Hz at 6dB per octave, and phantom power supplies 48V DC for compatible microphones, with a front panel LED indicating when it is active.

Audio and power are delivered via the EtherCON® connection, supporting PoE for simplified installation. The unit incorporates Audinate Dante chipsets and is compliant with AES67 and Dante Domain Manager.

Like all Sonifex DIO interfaces, it provides a simple plug-and-play solution for connecting analogue and digital audio equipment to Dante networks, with configuration handled via Dante Controller. It is housed in a rugged aluminium enclosure with provisions for desk mounting, and an optional accessory, AVN-DIORK, allows multiple units to be installed on a 19″ rack-mount shelf.

Typical Applications

To send any microphone signal to the Dante network, to amplify microphones in a studio talks area, court room microphone amplification, converting mic signal where mic and amp are a long distance apart.

Product Features

- 1 x balanced microphone input on XLR socket with latch lock.

- Neutrik EtherCon® Ethernet connection.

- Single turn pot setting fine mic gain (0dB – 36dB).

- Coarse mic gain switch (+20db/+50dB).

- High pass filter on/off switch.

- Phantom power on/off switch.

- Phantom power LED indicator.

- Level LED indicator.

- Fully Dante compliant device.

- AES67 compatible.

- Dante Domain Manager compliant.

- Ultra-high quality E.I.N. of >129dB.

- Powered via PoE (Power over Ethernet).

- Rack mount option with the AVN-DIORK.

- Aluminium construction.

Product Category

Microphone Amplifiers

TECHNICAL SPECIFICATIONS

Microphone XLR Input Pin-out

- Pin 1: Chassis ground

- Pin 2: Input phase

- Pin 3: Input non-phase

Earthing Point

- Earthing Point: Single earth tag

Level LED Thresholds

- Off: Level is under -38 dBFS (-20 dBu)

- Green: Level is between -38 dBFS (-20 dBu) and -18 dBFS (0 dBu)

- Amber: Level is between -18 dBFS (0 dBu) and -10 dBFS (+8 dBu)

- Red: Level is above -10 dBFS (+8 dBu)

Microphone Input – XLR

- Input Impedance: 2.2 kΩ balanced

- Maximum Input Level: -68 dBu (max gain) to -8 dBu (min gain)

- Gain: Adjustable 20 dB to 86 dB

- Frequency Response: 20 Hz to 20 kHz, +0/-0.5 dB (Ref. 1 kHz)

- High Pass Filter Response: Fc = 125 Hz @ 6 dB per octave

- THD+N: <0.01%, -32 dBu input, 40 dB gain, 20 Hz to 20 kHz, 20 kHz BW

- E.I.N.: 129 dBu, 20 kHz BW, max gain, Rs = 200 Ω

- Common Mode Rejection: >60 dB @ 1 kHz

- Phantom Power: +48 V ± 4 V

Dante

- Sample Rates: 44.1 kHz, 48 kHz, 88.2 kHz and 96 kHz

- Encoding: PCM 16, PCM 24 and PCM 32

PoE Power

- Standard: 802.3af

- Class: 0

- PD Power Range: 0.44 W to 12.94 W

- Typical PSE Power Usage: 4 W

- Max PSE Power Usage: 15.4 W

Physical Specification

- Dimensions (Raw): 10.6 cm (W) × 7.3 cm (D) × 4.3 cm (H)

- Dimensions (Raw): 4.2" (W) × 2.9" (D) × 1.7" (H)

- Dimensions (Boxed): 17.4 cm (W) × 9.5 cm (D) × 5.6 cm (H)

- Dimensions (Boxed): 6.9" (W) × 3.7" (D) × 2.2" (H)

- Weight: Nett: 0.2 kg. Gross: 0.3 kg

- Weight: Nett: 0.44 lbs. Gross: 0.66 lbs

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/avn-dio09/