AVN-DIO012

Dual Microphone Input to Dante® with Mic Gain

The AVN-DIO12 is a dual microphone input to Dante® interface with adjustable mic gain. It provides the same functionality as two AVN-DIO09 (single mic-to-Dante) units within a single chassis, while delivering high-performance A/D conversion with a world-class equivalent input noise (EIN) of 129dB.

The AVN-DIO12 is an ultra-high quality dual microphone to Dante® interface from the Sonifex DIO range, featuring two balanced analogue XLR inputs with high-performance mic preamps for seamless integration with Dante Audio-over-IP networks. It delivers A/D conversion with a world-class equivalent input noise (EIN) of 129dB.

Each microphone input includes independent coarse and fine gain adjustment, a switchable high-pass filter, phantom power, and a tri-colour level LED for accurate signal monitoring. Coarse gain is selectable between 14dB and 44dB via front panel switches, while fine gain provides an additional 0dB to 42dB of adjustment per channel.

The high-pass filter can be enabled to attenuate frequencies below 125Hz at 6dB per octave, and phantom power supplies 48V DC for compatible microphones, with front panel LEDs indicating when phantom is active on each channel.

Audio and power are delivered via the EtherCON® connection, supporting PoE for simplified installation. The unit incorporates Audinate Dante chipsets and is compliant with AES67 and Dante Domain Manager.

Like all Sonifex DIO interfaces, it provides a simple plug-and-play solution for connecting analogue and digital audio equipment to Dante networks, with configuration handled via Dante Controller. It is housed in a rugged aluminium enclosure with provisions for desk mounting, and an optional accessory, AVN-DIORK, is available to install multiple units on a 19″ rack-mount shelf.

Typical Applications

To send microphone signals to the Dante network and to route a mic signal over a long distance. Small and compact for easy placement. Distribute mic feeds over CAT5/6, at events, stadiums, court room or studio talks area.

Product Features

- Neutrik EtherCon® Ethernet connection.

- Sample rate conversion of physical inputs to Dante system sample rate.

- Single turn pots setting fine mic gain (0dB – 36dB).

- Coarse mic gain switches (+20db/+50dB).

- High pass filter on/off switches.

- Ultra-high quality, wide dynamic range A/D conversion.

- Phantom power on/off switches.

- Phantom power LED indicators.

- Level LED indicators.

- Fully Dante compliant device.

- AES67 compatible.

- Dante Domain Manager compliant.

- Powered via PoE (Power over Ethernet).

- Rack mount option with the AVN-DIORK.

- Aluminium construction.

Product Category

Microphone Amplifiers

TECHNICAL SPECIFICATIONS

Microphone XLR Input Pin-out

- Pin 1: Chassis ground

- Pin 2: Input phase

- Pin 3: Input non-phase

Microphone Input – XLR

- Input Impedance: 2.2 kΩ balanced

- Maximum Input Level: -68 dBu (max gain) to -8 dBu (min gain)

- Gain: Adjustable 20 dB to 86 dB

- Frequency Response: 20 Hz to 20 kHz, +0/-0.5 dB (Ref. 1 kHz)

- High Pass Filter Response: Fc = 125 Hz @ 6 dB per octave

- THD+N: <0.01%, -32 dBu input, 40 dB gain, 20 Hz to 20 kHz, 20 kHz BW

- E.I.N.: 129 dBu, 20 kHz BW, max gain, Rs = 200 Ω

- Common Mode Rejection: >60 dB @ 1 kHz

- Phantom Power: +48 V ± 4 V

Network and AoIP

- AoIP Standard: Dante

- Channels: 2 transmit

- Flows: 2 transmit

- Sample Rates: 44.1 kHz, 48 kHz, 88.2 kHz and 96 kHz

- Encoding: PCM 16, PCM 24 and PCM 32

- AES67 Support: Yes

- Connectivity: EtherCON (RJ45 compatible)

- Speed: 100 Mbps

- Dante Domain Manager Ready: Yes

- Clock Source: Internal (PTP Leader) or network PTP Leader

PoE Power

- Standard: 802.3af

- Class: 0

- PD Power Range: 0.44 W to 12.94 W

- Typical PSE Power Usage: TBD

- Max PSE Power Usage: 15.4 W

Physical Specification

- Dimensions (Raw): 14.0 cm (W) × 13.6 cm (D) × 4.2 cm (H)

- Dimensions (Raw): 5.5" (W) × 5.4" (D) × 1.7" (H)

- Dimensions (Boxed): 17.8 cm (W) × 17.0 cm (D) × 5.6 cm (H)

- Dimensions (Boxed): 7.0" (W) × 6.7" (D) × 2.2" (H)

- Weight: Nett: 0.5 kg. Gross: 0.7 kg

- Weight: Nett: 1.1 lbs. Gross: 1.4 lbs

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/avn-dio12/