AVN-DIO16

Dante® to 4 Analogue XLR Outputs

The AVN-DIO16 is a 4 analogue XLR output to Dante converter. It features 4 balanced analogue XLR outputs and one Neutrik etherCON connector for direct connection to a Dante AoIP network.

The AVN-DIO16 is an ultra-high-quality Dante-to-analogue converter from the Sonifex DIO range. It features four balanced analogue XLR outputs for seamless integration with Dante Audio-over-IP networks. A front panel line-up control allows the global 0dBFS reference level to be set to +12dBu, +18dBu, or +24dBu using a recessed toggle switch, enabling flexible system matching. The superior D/A circuitry used is designed for optimal audio performance, offering >120dB of dynamic range. Audio and power are delivered via the EtherCON® connection, supporting PoE for simplified installation. Front panel LEDs indicate network clock status, AoIP link activity, and PoE power presence for clear operational monitoring. The unit incorporates Audinate Dante chipsets and is compliant with AES67 and Dante Domain Manager.

Like all Sonifex DIO interfaces, it provides a simple plug-and-play solution for connecting analogue and digital audio equipment to Dante networks, with configuration handled via Dante Controller. It is housed in a rugged aluminium enclosure with provisions for desk mounting, and an optional accessory, AVN-DIORK, allows multiple units to be installed on a 19″ rack-mount shelf.

Typical Applications

The AVN-DIO16 is suited to a wide range of professional audio environments where reliable, low channel count, low-latency transport between analogue and networked systems is required. It is ideal for broadcast studios, live production, events, and small fixed AV installations such as corporate, education, residential and hospitality spaces. It also integrates seamlessly into houses of worship and recording studios, providing flexible, high-quality connectivity between traditional analogue equipment and Dante-based audio networks.

Product Features

- 4 x balanced analogue outputs on XLR.

- Ultra-high quality, wide dynamic range D/A conversion, >120dB.

- Adjustable global 0dBFS line up selection (+12dBu/+18dBu/+24dBu).

- 1 x Neutrik etherCON® connector (1Gb/s Ethernet Port).

- PoE, Link, and Clock LED status indicators.

- Configuration using Dante Controller.

- AES67 compatible.

- Dante Domain Manager compliant.

- Powered via PoE (Power over Ethernet).

- 1U rack-mount option with the AVN-DIORK.

- Aluminium construction.

TECHNICAL SPECIFICATIONS

Analogue XLR Input / Output Pin-out

- Pin 1: Chassis Ground

- Pin 2: Phase

- Pin 3: Non Phase

Line Outputs – XLR

- Output Impedance: <200Ω Balanced

- 0dBFS Line-Up: Selectable +12dBu / +18dBu / +24dBu

- Frequency Response: 20Hz to 20kHz, +0/-0.5dB (ref. 1kHz)

- THD+N: <-100dBu, -30dBFS, 20Hz to 20kHz, 20kHz BW

- Dynamic Range: 120dB, 20kHz BW

- Cross Talk: <-110dB

Network and AoIP

- AoIP Standard: Dante

- Number of Channels: 4 Receive

- Number of Streams: 2 Receive

- Sample Rates: 44.1kHz, 48kHz, 88.2kHz, 96kHz

- Encoding Format: Linear PCM, 16, 24 or 32-bit

- AES67 Support: Yes

- Connectivity: etherCON (RJ45 compatible)

- Speed: 100Mbps

- Dante Domain Manager Ready: Yes

- Clock Source: Internal (PTP Leader) or Network PTP Leader

PoE Power

- Standard: IEEE 802.3af

- Class: 0

- PD Power Range: 0.44W to 12.94W

- Typical PSE Power Usage: 6W

- Maximum PSE Power Usage: 15.4W

Physical Specification

- Dimensions (Nett): 14.0cm (W) × 13.6cm (D) × 4.2cm (H)

- Dimensions (Nett): 5.5" (W) × 5.4" (D) × 1.7" (H)

- Dimensions (Gross): 17.8cm (W) × 17.0cm (D) × 5.6cm (H)

- Dimensions (Gross): 7.0" (W) × 6.7" (D) × 2.2" (H)

- Weight (Nett): 0.42kg (0.95lbs)

- Weight (Gross): 0.56kg (1.25lbs)

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/avn-dio16/