AVN-DIO19

Dante® to AES3 16 Channel I/O Converter

The AVN-DIO19 audio converter and interface converts up to eight digital stereo AES3 inputs and eight digital stereo AES3 outputs to and from the Dante Audio-over-IP networking standard.

The AVN-DIO19 is an AES3 to Dante audio converter, supporting up to eight stereo AES3 inputs and eight stereo AES3 outputs for seamless integration with Dante Audio-over-IP networks.

Each input accepts sample rates from 32kHz to 192kHz and is automatically sample rate converted to match the Dante system rate. All outputs operate at the system sample rate, ensuring consistent performance across the network. Optional SRC bypass to AES inputs is also available.

Front panel LEDs provide clear status indication, including network clock, primary and secondary link activity, and PoE power presence. Additional LEDs show lock status for all eight AES3 inputs.

Audio and power are delivered via the EtherCON® connection. Primary and secondary ports provide power and data redundancy, supporting PoE for simplified installation. Front panel LEDs indicate network clock status, AoIP link activity, and PoE power presence for clear operational monitoring. The unit incorporates Audinate Dante chipsets and is compliant with AES67 and Dante Domain Manager. A built-in web interface allows access to device configuration, network settings, status monitoring, and firmware updates.

Like all Sonifex DIO interfaces, it provides a simple plug-and-play solution for connecting analogue and digital audio equipment to Dante networks, with configuration handled via Dante Controller. It is housed in a rugged aluminium enclosure with provisions for desk mounting, and an optional accessory, AVN-DIORK, allows multiple units to be installed on a 19″ rack-mount shelf.

Typical Applications

The AVN-DIO19 is suited to a wide range of professional audio environments where reliable, low-latency transport between digital and networked systems is required. It is ideal for broadcast studios, live production and events, OB trucks, and fixed AV installations such as corporate, education, and hospitality spaces. It also integrates seamlessly into houses of worship and recording studios, providing flexible, high-quality connectivity between traditional digital equipment and Dante-based audio networks.

Product Features

- 8 x balanced digital stereo AES3 inputs and outputs on 2 x 25-way D-types.

- Sample rate conversion of physical inputs to Dante system sample rate.

- Physical output sample rate matches Dante system sample rate.

- Optional SRC bypass to AES inputs.

- All available Dante sample rates supported.

- Dual redundant Primary and Secondary Dante network ports using Neutrik EtherCON® Ethernet connectors.

- Powered via PoE (Power over Ethernet) with PoE dual redundancy.

- Fully Dante compliant device.

- Dante Domain Manager compliant.

- AES67 compatible.

- Web interface for configuration.

- Clock, PoE, Network link and AES3 input lock LEDs.

- 1U rack-mount option with the AVN-DIORK.

- Aluminium construction.

TECHNICAL SPECIFICATIONS

AES3 I/O

- Input & Output Impedance: 110Ω Balanced

- Sample Rates: 32kHz to 192kHz

Network and AoIP

- AoIP Standard: Dante

- Number of Channels: 16 Receive, 16 Transmit

- Number of Streams: 16 Receive, 16 Transmit

- Sample Rates: 44.1kHz, 48kHz, 88.2kHz, 96kHz, 176.4kHz, 192kHz

- Encoding Format: Linear PCM, 16, 24 or 32-bit

- Connectivity: 2 × etherCON (RJ45 compatible)

- Speed: 1Gbps or 100Mbps

- Network Modes: Switched or Redundant

- AES67 Support: Yes

- Dante Domain Manager Ready: Yes

- Clock Source: Internal (PTP Leader) or Network PTP Leader

PoE Power

- Standard: IEEE 802.3af

- Redundancy: Yes

- Class: 0

- PD Power Range: 0.44W to 12.94W

- Typical PSE Power Usage: 4.6W

- Maximum PSE Power Usage: 15.4W

Physical Specification

- Dimensions (Nett): 14.0cm (W) × 13.6cm (D) × 4.2cm (H)

- Dimensions (Nett): 5.5" (W) × 5.4" (D) × 1.7" (H)

- Dimensions (Boxed): 17.8cm (W) × 17.0cm (D) × 5.6cm (H)

- Dimensions (Boxed): 7.0" (W) × 6.7" (D) × 2.2" (H)

- Weight (Nett): 0.5kg (1.1lbs)

- Weight (Gross): 0.9kg (2.0lbs)

Accessories

- AVN-DIORK: AVN-DIO 1U 19" Rack Kit (supports 5 × small DIO or 3 × large DIO)

- CBL-D25-4XI4XO: AES3 balanced cable, DB25 to 4 × XLR3M and 4 × XLR3F, 3m

- CBL-D25-D25: AES3 balanced cable, DB25 to DB25, 3m

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/avn-dio19/