AVN-DIO20

Dante® to MADI AES3 64 Channel I/O Converter

The AVN-DIO20 is a MADI and AES3 to Dante bridging device allowing MADI to connect bidirectionally with AoIP, together with 8 stereo channels of AES3.

The AVN-DIO20 is a MADI and AES3 to Dante bridging device that enables bidirectional connection between MADI and AoIP, alongside 8 stereo AES3 channels. This plug-and-play interface provides a quick and convenient method for integrating legacy MADI equipment into a Dante audio network.

The unit accepts a MADI input, sample rate converts all 64 channels, and places them onto the network, where routing is managed via Dante Controller. At the same time, it receives 64 channels from the network, optionally applies sample rate conversion, and outputs them as MADI.

It also supports 8 stereo AES3 inputs, which are mapped onto the network and replace 16 selected MADI input channels. Additionally, 8 stereo AES3 outputs can be sourced from network channels in blocks of 16.

MADI input is available via either a compatible SFP module or a coaxial BNC connection. Input selection can be configured through the web GUI, or defaults to the SFP when present. The BNC output is disabled while the SFP is in use. An automatic failover mode enables seamless switching to the BNC input if the SFP signal is lost, ensuring glitch-free redundancy. The MADI output can be clocked either from the network (bypassing output SRC) or from the recovered MADI input clock.

Audio and power are delivered via the EtherCON® connection. Primary and secondary ports provide power and data redundancy, supporting PoE for simplified installation. Front panel LEDs indicate network clock, primary and secondary link status, PoE power activity, AES3 input lock (8 channels), and active MADI input presence, all for clear operational monitoring. The unit incorporates Audinate Dante chipsets and is compliant with AES67 and Dante Domain Manager. A built-in web interface allows access to device configuration, network settings, status monitoring, and firmware updates.

Like all Sonifex DIO interfaces, it provides a simple plug-and-play solution for connecting analogue and digital audio equipment to Dante networks, with configuration handled via Dante Controller. It is housed in a rugged aluminium enclosure with provisions for desk mounting, and an optional accessory, AVN-DIORK, allows multiple units to be installed on a 19″ rack-mount shelf.

Typical Applications

Converting legacy MADI connections to the Dante network, perfect for mixing consoles with MADI connectivity. Converting MADI to Dante in OB trucks, together with AES3 conversion for AV installations.

Product Features

- Dante AoIP.

- 1 x AES10 MADI input and output, on either SFP or coaxial BNC (up to 64 channels of MADI I/O).

- 8 x stereo AES3 inputs and outputs on 2 x 25-way D-types, using AES59 digital pinout.

- Dual redundant Primary and Secondary Dante network ports using Neutrik EtherCON® Ethernet connectors.

- Powered via PoE (Power over Ethernet) with PoE dual redundancy.

- Clock, PoE, Network link, AES3 input lock and MADI active LEDs.

- MADI 64, 56, 28 and 16 channel support and audio sample rate support up to 192kHz.

- MADI SFP to BNC automatic failover.

- Optional SRC bypass to AES inputs.

- Fully Dante compliant device.

- Dante Domain Manager compliant.

- Dante clock domain can be optionally synchronised from the MADI source.

- Sample rate conversion of audio between Dante and MADI/AES3.

- All available Dante sample rates supported (44.1kHz to 192kHz).

- AES67 compatible.

- Web interface for configuration.

- 1U rack-mount option with the AVN-DIORK.

- Aluminium construction.

TECHNICAL SPECIFICATIONS

MADI I/O

- BNC Input & Output Impedance: 75Ω Unbalanced

- MADI Input Sample Rate / Channels:

  - 48kHz – 56/64 channels

  - 96kHz – 28/32 channels

  - 192kHz – 16 channels

- MADI Output Sample Rate / Channels:

  - Follows MADI input

  - Dante sample rate

- Connections:

  - Coaxial BNC input and output

  - SFP – LVDS SFP 100MB/s

AES3 I/O

- Input & Output Impedance: 110Ω Balanced

- Sample Rates: 32kHz to 192kHz

- Connections: 8 × stereo AES3 inputs and outputs on 2 × 25-way D-type connectors using AES59 pinout

Network and AoIP

- AoIP Standard: Dante

- Number of Channels: 64 Receive, 64 Transmit

- Number of Streams: 32 Receive, 32 Transmit

- Sample Rates: 44.1kHz, 48kHz, 88.2kHz, 96kHz, 176.4kHz, 192kHz

- Encoding Format: Linear PCM, 16, 24 or 32-bit

- Connectivity: 2 × etherCON (RJ45 compatible)

- Speed: 1Gbps or 100Mbps

- Network Modes: Switched or Redundant

- AES67 Support: Yes

- Dante Domain Manager Ready: Yes

- Clock Source: Internal (PTP Leader), Network PTP Leader or MADI Input

PoE Power

- Standard: IEEE 802.3af

- Redundancy: Yes

- Class: 0

- PD Power Range: 0.44W to 12.94W

- Typical PSE Power Usage: 6.1W

- Maximum PSE Power Usage: 15.4W

Physical Specification

- Dimensions (Nett): 14.0cm (W) × 13.6cm (D) × 4.2cm (H)

- Dimensions (Nett): 5.5" (W) × 5.4" (D) × 1.7" (H)

- Dimensions (Boxed): 17.8cm (W) × 17.0cm (D) × 5.6cm (H)

- Dimensions (Boxed): 7.0" (W) × 6.7" (D) × 2.2" (H)

- Weight (Nett): 0.4kg (0.9lbs)

- Weight (Gross): 0.6kg (1.3lbs)

Accessories

- AVN-DIORK: AVN-DIO 1U 19" Rack Kit (supports 5 × small DIO or 3 × large DIO)

- CBL-D25-4XI4XO: AES3 balanced cable, DB25 to 4 × XLR3M and 4 × XLR3F, 3m

- CBL-D25-D25: AES3 balanced cable, DB25 to DB25, 3m

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/avn-dio20/