CM-HPR1

Headphone Volume Control

This product allows the connection of a stereo balanced line input via a RJ45 with StudioHub+™ pinout and creates a headphone output, with level control. It is transformer balanced, and can be used with any headphones above 150Ω impedance. The transformer ratio of 2:1 ensures that any headphone load presented at the output is reflected through to the input at a 1:4 ratio (hence a 150Ω set of headphones looks like 600Ω at the input, a suitable load for most good quality line output stages).

The stereo headphone output uses a stereo 6.35mm jack socket with the following connections:

Pin Type & Connection:

Tip: Left.
Ring: Right.
Screen: Ground.
The headphone level will be at half the level of the incoming signal with the volume control at 100%.

The volume level control sets the output level of the stereo headphone socket and runs logarithmically from zero to the maximum output level.

The stereo analogue input is on a female RJ45 connector with the following standard StudioHub+™pin assignments.

Connection:

1 Left Phase.
2 Left Non-Phase.
3 Right Phase.
4 Ground.
5 N/C.
6 Right Non-Phase.
7 N/C.
8 N/C.

Typical Applications

Creates a connection of a stereo balanced line input via a RJ45 with StudioHub+™ pinout and provides a headphone output, with level control.

Product Features

- Stereo line level RJ45 (StudioHub+™ pin-out).

- Headphone output with level control.

- For headphones above 150Ω impedance.

TECHNICAL SPECIFICATIONS

Audio Specification

- Input/Output Level Ratio: 02:01

- Input/Output Impedance Ratio: 04:01

- Minimum Headphone Impedance: 150Ω

- Max Input Level: +28dBu

- Max Output Level: +22dBu

- Frequency Response: 20Hz to 20kHz, +0/-0.5dB

- Noise: <-115dBu A-wtd

- Distortion (Ref +8dBu Output): <0.01%

- Common Mode Rejection: >80dB

Connections

- Main Stereo Input: 1 × RJ45 female (StudioHub+™ pinout)

- Output: 1 × ¼” (6.35mm) 3-pole stereo jack sockets

Physical Specification

- Dimensions (Raw): 7.7cm (W) x 8.3cm (D) x 4.2cm (H)

- Dimensions (Raw): 3.0” (W) x 3.3” (D) x 1.7” (H)

- Weight: Nett 0.22kg, Gross 0.33kg

- Weight: Nett 0.49lbs, Gross 0.73lbs

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/cm-hpr1/