CM-LUX1

Balanced to Unbalanced Audio Converter

This product allows the connection of a stereo balanced line input via 2 x female XLRs to a stereo unbalanced output on phono connectors.

It is transformer balanced, with a 2:1 ratio, so 6dB of attenuation is added to the input to help interface with the lower signal levels associated with consumer outputs.

This unit is a passive version of the RB-LU4 (though only 1 x stereo channel), and provides you with an alternative to having to interface a balanced output to an unbalanced input via specialist cables (either by shorting the non-phase to ground or disconnecting it).

It is particularly useful where a shared balanced audio feed needs interfacing to an unbalanced input on one or more pieces of equipment. Using a CM-LUR1 or CM-LUX1 galvanically isolates the audio, allowing you to connect via phono cables to an unbalanced input without compromising the distributed feed.

Analogue Inputs

The stereo analogue input is on two female 3 pin XLR connectors with the following pin assignments:
Pin 1: 0V Common
Pin 2: Phase
Pin 3: Non-Phase

RCA Phono Analogue Outputs

This unit uses a stereo unbalanced line output on a RCA phono connector with the following connections:

Pin Type – Connection
White Tip: Left
White Screen: Ground
Red Tip: Right
Red Screen: Ground

Typical Applications

Creates a connection of a stereo balanced line input via 2 x female XLRs to a stereo unbalanced output on phono connectors.

Product Features

- Stereo balanced line input via XLR-3.

- Stereo unbalanced output on stereo phono connectors.

- Transformer-balanced input with 2:1 ratio.

TECHNICAL SPECIFICATIONS

Audio Specification

- Input/Output Level Ratio: 02:01

- Input/Output Impedance Ratio: 04:01

- Max Input Level: +28dBu

- Max Output Level: +22dBu

- Frequency Response: 20Hz to 20kHz, +0/-0.5dB

- Noise: <-115dBu A-wtd

- Distortion (Ref +8dBu Output): <0.01%

- Common Mode Rejection: >80dB

Connections

- Balanced Line Input: 2 × female XLR connectors

- Unbalanced Line Output: 2 × RCA phono connectors

Physical Specification

- Dimensions (Raw): 7.7cm (W) x 8.3cm (D) x 4.2cm (H)

- Dimensions (Raw): 3.0” (W) x 3.3” x 1.7” (H)

- Weight: Nett 0.22kg, Gross 0.33kg

- Weight: Nett 0.49lbs, Gross 0.73lbs

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/cm-lux1/