CM-BH4W

Belt-Pack 4-Wire Headphone Amp

The CM-BH4W is a portable, battery powered 4 wire communication unit for use in a multitude of TV and radio broadcasting applications. It has a 1/4″ (6.35mm) headset connector.

The CM-BH4W is a portable, battery powered 4 wire communication unit for use in a multitude of TV and radio broadcasting applications. This uses audio transformers at its input and output, providing galvanic isolation from outside interference, ensuring your communications are loud and clear, every time.

In 4-wire mode, the unit acts as a 2-way intercom, with input audio routed to the headset earpiece, and microphone audio is routed to the output upon pressing the talk button. The output level is maintained at +2dBu peak level by a built-in AGC.

In IFB mode, the input audio is routed to the output, and is interrupted by the microphone signal when the talk switch is pressed. The talk switch can easily be converted from momentary to latching operation to suit your application.

A low battery warning LED flashes when the 9V PP3 battery has depleted to 6V, giving plenty of notice to swap the battery. In addition, the CM-BH4W automatically switches off when the headset jack is removed, and the low power circuitry gives up to 150 hours use from a single alkaline 9V PP3 battery.

Typical Applications

Used in TV studio floors, sports grounds and arenas by presenters and cameramen to produce content and to hear talkback.

Product Features

- Transformer balanced inputs & outputs.

- 4-wire and IFB modes.

- Low battery warning indicator.

- 150 hours from a single 9V PP3 battery.

TECHNICAL SPECIFICATIONS

Mic In to Line Out

- Mic Input Impedance: 1.5kΩ unbalanced for 200Ω dynamic microphone

- Limiter Input Threshold: -60dBu

- Line Output Level: +2dBu

- Line Output Impedance: 150Ω transformer balanced

Line In to Headphone Out

- Line Input Impedance: >10kΩ transformer balanced

- Input Gain/Volume: Adjustable from off to +10dB

- Maximum Input Level: +24dBu

- Maximum Headphone Output Level: +7dBu (ref. 9V battery voltage, <1% THD, 100Ω load)

- Minimum Headphone Impedance: 100Ω

Connections

- Line Input: 1 × XLR 3-pin female socket (balanced)

- Line Output: 1 × XLR 3-pin male plug (balanced)

- Headset (CM-BH4W): 1 × ¼" (6.35mm) mono unbalanced jack socket

Physical Specification

- Dimensions (Raw): 12.5cm (W) × 6.7cm (D) × 4.8cm (H) / 4.9" (W) × 2.6" (D) × 1.9" (H)

- Dimensions (Boxed): 23cm (W) × 13cm (D) × 8cm (H) / 9.1" (W) × 5.1" (D) × 3.1" (H)

- Weight: Nett: 0.3kg (0.6lbs), Gross: 0.4kg (1.0lbs)

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/cm-bh4w/