CM-CU1

Commentator Unit 1 Commentator & Line Input

The CM-CU1 offers the same high quality, robust specification as the CM-CU21, with a feature set tailored to single commentator applications.

The CM-CU1 offers the same high quality, robust specification as the CM-CU21, tailored to single commentator applications. It provides a fully featured commentator position and a line-level input. The unit has an individual commentator output, with an additional output providing a mix of commentator and line input audio. A limit indicator on the main panel shows when the adjustable limiter on the main output is active, and a bright 21 segment LED PPM meter, which can be disabled, shows the main output level.

The line input can be configured to remain present at the mix output even if the commentator is off air, making it useful for routing crowd effects or pre-recorded material to the programme feed. Its presence in the commentator’s headphones is also configurable to suit the application.

Four talkback output channels, with a built-in limiter, are available to the commentator and they can be linked to provide simultaneous operation. Activation of one or more talkback channels removes the commentator audio from the main output until all talkback channels are deactivated.

A wide input gain range and switchable phantom power allows support of a variety of microphone types including low output ribbon mics, headsets and high output phantom powered microphones. There are 5 transformer balanced return audio inputs which, along with the commentator input, are routed to the headphone monitor controls. With these headphone controls, any monitor input can be adjusted and sent to either the left, right or both ears.

A GPIO port gives an open-collector driven output indication of any active On-Air or talkback channel and four remote inputs provide an external call/alert to each of the talkback controls. The unit has a built-in line-up tone generator for easy channel identification and cabling checks. When enabled the test-tone is routed to the required output simply by pressing the corresponding on air or talkback button. An automatic tone mode repeatedly sends different quantities of tone bursts to each output to make it effortless for a single technician to set up.

An On-Air lock switch prevents the On-Air button from being accidentally deactivated and a T/B Gang switch links the operation of talkbacks A, B, C & D. A flexible range of options allow the Commentator Unit to be customised to better suit the operational and user’s personal requirements. These include options for momentary or latching operation of the talkback buttons, permanently enabling or disabling the commentator input, default headphone signal routing and main panel indicator illumination levels. The CM-CU1 Commentator Unit can be powered from a universal mains input between 85-264V AC at 47-63Hz, or from a 12-24V DC input, both on the rear panel. A transport case, CM-TC1 – with unit cut-outs, and the CM-TC2 – without cut-outs, is also available to protect the unit.

Typical Applications

Used in sports grounds and arenas by commentators to produce commentary & to hear talkback.

Product Features

- Commentator position with individual output; plus main mix output.

- Line input with configurable output and headphone presence.

- Talkback outputs selectable by the commentator, with built in limiter and adjustable gain.

- 5 Return audio circuits that are routed to headphones along with the commentator input. Any source can be individually level controlled and sent to left, right or both ears.

- A wide input gain range and switchable phantom power allows support of a variety of microphone types including low output ribbon, headsets and high output phantom powered microphones.

- User preset adjustable limiter on main output with limit activity indication.

- Built in line-up tone for initial cabling checks.

- 21 Segment LED PPM meter showing main output level.

- Transformer balanced inputs & outputs.

- Fitted formica ‘Sharpie’ designation strips.

- Robust mechanical design with protection for potentiometer knobs.

- Universal AC mains, or 12–24-volt DC power.

TECHNICAL SPECIFICATIONS

Microphone Input

- Gain Range: +20dB to +86dB

- Maximum Input Level: -6dBu

- Equivalent Input Noise: 130dB (Ref. 80dB gain with 150Ω termination)

- Equivalent Input Noise: 128dB (Ref. 50dB gain with 150Ω termination)

- C.M.R.R.: >60dB

- Frequency Response: ±0.5dB, 20Hz to 22kHz (Ref. 50dB gain @ 1kHz)

Electronically Balanced Line Input

- Gain Range: -83dB to +10dB

- Maximum Input Level: +25dBu

- C.M.R.R.: >70dB

- Frequency Response: ±0.2dB, 20Hz to 22kHz (Ref. 0dBu @ 1kHz)

Transformer Balanced Monitor Inputs

- Gain Range: -66dB to +11dB (measured at headphone output)

- Maximum Input Level: +21dBu

- C.M.R.R.: >55dB

- Frequency Response: ±0.5dB, 20Hz to 22kHz (Ref. 0dBu @ 1kHz)

Transformer Balanced Mix Output

- Maximum Output Level (Limiter Off): +24dBu

- Frequency Response: ±0.5dB, 20Hz to 22kHz (Ref. 0dBu @ 1kHz)

- Noise: -94dB average weighting (Ref. +8dBu)

- THD & N: <0.02% (Ref. 1kHz @ +8dBu)

- THD & N: <0.065% (Ref. 1kHz @ +18dBu)

- 1% Distortion Point: +18dBu @ 24Hz

Transformer Balanced Talkback Outputs

- Maximum Output Level (Limited): +8dBu

- Frequency Response: ±0.5dB, 20Hz to 22kHz (Ref. 0dBu @ 1kHz)

- Noise: -94dB average weighting (Ref. +8dBu)

- THD & N: <0.02% (Ref. 1kHz @ +8dBu)

- Gain Adjustment: 0dB to +12dB

Electronically Balanced Comm Output

- Maximum Output Level: +25dBu

- Frequency Response: ±0.2dB, 20Hz to 22kHz (Ref. 0dBu @ 1kHz)

- Noise: -98dB average weighting (Ref. +8dBu)

- THD & N: <0.002% (Ref. 1kHz @ +8dBu)

Headphone Output

- Gain Range: -66dB to +11dB

- Maximum Output Level: +19dBu

- Frequency Response: ±0.5dB, 20Hz to 22kHz (Ref. 0dBu @ 1kHz)

- Noise: -85dB average weighting (Ref. +8dBu)

- THD & N: <0.005% (Ref. 1kHz @ +8dBu)

Main Panel Operational Controls & Indicators

- On-Air Button: Illuminated button with latching action

- Talkback A, B, C & D Buttons: Illuminated buttons with selectable momentary and/or latching action

- Headphone Monitor Controls: Input source level adjustment potentiometers

- Headphone Monitor Controls: Headphone channel selection switches

- Line Input Level: Line input mode level potentiometer

- PPM Meter: 21-segment display showing mix output level with selectable brightness

- Limit LED: Indicates main output limiter is active, colour and brightness selectable

- Power LED: Indicates operating mode (normal or setup)

Front Panel Operational Controls

- Phantom: Switch to enable 48V microphone phantom power

- Mic Gain: Gain potentiometer with range select switch (+20dB to +56dB or +50dB to +86dB)

Rear Panel Operational Controls

- On Air Lock: Switch to prevent On-Air control from being deactivated

- On Air Lock Override: Momentary push-button to override On-Air lock

- T/B Gang: Switch to link operation of Talkback A, B, C & D

- T/B Gain: 0dB to +12dB

- Limiter: Switch to enable mix output limiter

- Limiter Level: Limit level adjustment potentiometer

- Test Tone: Switch to enable 1kHz line-up tone

Configuration Options – Accessible via Setup Mode

- On Air Control Mode: Disabled, normal or permanently on

- Talkback Button Action: Momentary, latching or both

- Headphone Routing: Line-level input permanently routed or routed only when On-Air to commentator headphones

- Headphone Routing: Commentator input permanently routed or not routed to headphone monitors

- Line Level Input Routing: Permanently routed, not routed or routed only when On-Air to main output

- Display Preferences: Limit LED colour and brightness

- Display Preferences: Button LED brightness

- Display Preferences: PPM meter LED brightness or disable

- Permanent Option Backup: Save/load options to permanent backup or restore factory defaults

Front Panel Connections

- Headphone Output: 1 x locking headphone jack socket

- Microphone Input: 1 x XLR-3 pin latching female

- Line Input: 1 x XLR-3 pin latching female

Rear Panel Connections

- Program, A, B, C & D Inputs: 5 x XLR-3 pin female (transformer balanced)

- Comm Output: 1 x XLR-3 pin male (electronically balanced)

- Talkback A, B, C & D Outputs: 4 x XLR-3 pin male (transformer balanced)

- Mix Output: 1 x XLR-3 pin male (transformer balanced)

- GPIO Port: 15-way D-type socket

- Mains Input: Universal filtered IEC, 85–264VAC, 47–63Hz, 10W

- Fuse Rating: 1A anti-surge fuse, 20 x 5mm

- DC Input: XLR-4 pin male, 12V DC (650mA typical, 850mA maximum)

- DC Input: 24V DC (325mA typical, 425mA maximum)

- Earth Point: M4 stud

Physical Specification

- Dimensions (Raw): 270mm (W) x 230mm (D) x 70mm (H - front) x 95mm (H - rear)

- Dimensions (Raw): 10.6" (W) x 9" (D) x 2.8" (H - front) x 3.7" (H - rear)

- Dimensions (Boxed): 380mm (W) x 350mm (D) x 200mm (H)

- Dimensions (Boxed): 15" (W) x 13.8" (D) x 7.8" (H)

- Weight: Nett 3.3kg, Gross 4.3kg

- Weight: Nett 7.3lbs, Gross 9.4lbs

Accessories

- CM-CUTC: Commentator Unit Transport Case

- CM-GM2: Professional Gooseneck Condenser Microphone

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/cm-cu1/