PS-SENDS-SD

The PS-SENDS-SD is a 19” rack-mount audio to IP converter which is also available in a freestanding unit, the PS-SEND-SD.

The PS-SENDS-SD is a 19” rack-mount audio to IP converter. It receives audio from a number of user selectable external stereo sources including balanced and unbalanced analogue audio, AES/EBU, S/PDIF & TOSlink digital audio. Once an audio source is selected, the unit encodes the audio in real time and sends it to the network as an encoded stream.  The audio stream can be distributed over an IP-based network to one or more PS-PLAY or PS-AMP units or other proprietary servers such as those for Icecast or Shoutcast. All the configuration settings for the unit are accessed via a local web-server built into it. The type of encoding and the transport mechanism are defined by selecting the connection from a pre-defined list. The PS-SEND-SD encodes an audio source into an MP3 (from analogue or digital inputs), G.711 or PCM (from analogue inputs only) audio stream using HTTP, RTP, raw UDP or raw TCP protocols, including multicast support and the following encoder types are available: Mpeg1 & 2 Layer3, MP3 CBR (constraint bit-rate), PCM linear and A-law, U-law, with 8kHz-48kHz sample rates. The unit can configure its own IP address using DHCP/BOOTP, IPzator or AutoIP. A readout of the set IP address can be heard on every reset using SONICIP technology, if selected. Remote level monitoring is also possible using SNMP traps.

Two red and green front panel LEDs indicate what state the unit is currently in, be it normal operational mode or bootstrap mode, and also indicates the current network connection status. A blue LED denotes power to the unit. The input being routed to the IP stream can be monitored on the front panel ¼” (6.35mm) stereo jack socket in combination with a headphone volume knob. The rear panel has 2 x RJ45 connectors, one for the 10/100Mbit Ethernet interface and one for GPI connections. The PS-SEND-SD has 6 x GPIs which can be used to trigger the sending of the audio stream and which can also be used to trigger remote events using an output relay on the PS-PLAY and PS-AMP. There is a 9-way D-type RS232 serial connection for control of the unit by automation systems and firmware updates. The unit can be remote controlled via serial connection, TCP or UDP. Power to the unit is via a universal supply 85V – 264V fused IEC mains socket.

Typical Applications

- As a backup STL (studio to transmitter link).

- For audio confidence monitoring in remote locations, such as at a radio transmitter site.

- For distribution of audio and music around a building, such as for passing audio to speakers in a conference room.

- As an internet-based IP music distribution system.

- As a tannoy, paging or IP based public address system.

- For in-house audio applications and distribution.

- For streaming internet radio in bars and clubs.

- For radio and music channels in hotel rooms distributed via an IP network.

- For playing audio from a PC, a jukebox application, or from a USB stick.

Product Features

- Audio to IP streamer.

- Built-in web server for configuration.

- Balanced & unbalanced analogue audio inputs.

- AES/EBU, S/PDIF and TOSlink digital audio inputs.

- Front panel headphone monitor with volume control.

- 6 x GPI for triggering remote events.

TECHNICAL SPECIFICATIONS

Technical Specification

- Analogue Inputs: 2 × XLR 3-pin (balanced, L & R)

- Analogue Inputs: 2 × RCA phono (unbalanced, L & R)

- Maximum Analogue Input Level: +18dBu (XLR balanced), +8dBu (RCA phono unbalanced)

- Balanced Input Impedance: 20kΩ

- Analogue Input SNR: 74dB

- Frequency Response: 20Hz–20kHz (using MP3 at a bit rate of 192kbps and above)

- Input THD: 0.02% relative

- Interchannel Isolation (Crosstalk): 80dB (Ref FSD)

- Digital Inputs: 1 × AES/EBU XLR 3-pin female

- Digital Inputs: 1 × S/PDIF RCA phono

- Digital Input: 1 × TOSLink optical

- Analogue Output: 1 × ¼" headphone jack

- Headphone Output: Drives 150mW into 32Ω to 600Ω stereo headphones

- GPIs (General Purpose Inputs): 6 × GPIs, configurable via the web interface using the RJ45 connection

- Serial Port: 1 × 9-pin D-type socket for control commands and firmware updates

- Ethernet Port: 1 × RJ45 socket supporting remote control via TCP/UDP and firmware updates

- Mains Input: Filtered IEC, 85–264VAC, 47–63Hz, 10W max

- Fuse Rating: 1A anti-surge, 20 × 5mm

Audio Codec Specifications

- G.711: μ-Law/A-Law, 8kHz–48kHz sampling rate

- WAV: IMA ADPCM+ and 16-bit PCM uncompressed, 8kHz–48kHz

- MP3 MPEG-1 Layer III: 32kHz, 44.1kHz and 48kHz (CBR)

- MP3 MPEG-2 Layer III: 16kHz, 22.05kHz and 24kHz (CBR)

Supported Network Transport Protocols

- RTP: UDP

- HTTP: TCP

- Raw UDP

- Raw TCP

- Icecast/Shoutcast: Can act as an Icecast/Shoutcast source

- SNMP: Trap support for remote management

- Dynamic IP Address Resolution: DHCP, BOOTP, IPZator or AUTOIP

- SonicIP: IP address readout

Physical Specification

- Dimensions (Raw): 48.3cm (W) × 13.7cm (D) × 4.3cm (H) / 19" (W) × 5.39" (D) × 1.7" (H)

- Dimensions (Boxed): 58.8cm (W) × 27cm (D) × 6.8cm (H) / 23" (W) × 10.6" (D) × 2.7" (H)

- Weight: Nett: 1.34kg (2.95lbs), Gross: 2.14kg (4.7lbs)

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/ps-sends-sd/