PS-PLAY

IP to Audio Streaming Decoder

The PS-PLAY is a freestanding IP to audio converter which is also available in a 1U rack-mount as the PS-PLAYS.

The PS-PLAY is a freestanding IP to audio converter. It takes an IP audio feed and converts it to a number of simultaneous stereo outputs: balanced and unbalanced analogue audio, AES/EBU, S/PDIF & TOSlink digital audio outputs.

As for the PS-SEND, all the configuration settings for the unit are accessed via a local web-server built into it. The unit can decode one of a number of audio streams, such as those generated by the PS-SEND (MP3, G.711 and PCM) and including Ogg Vorbis audio files from external USB as well as from sources such as Shoutcast, Icecast (Internet radio), VLC and from RTP servers. The unit can receive streams from HTTP (TCP/IP) and  RTP (UDP) protocols, as well as raw TCP and UDP packets. The unit can also configure its own IP address using DHCP/BOOTP, IPzator or AutoIP. A readout of the set IP address can be heard on every reset using SONICIP technology, if selected.

Two red and green front panel LEDs indicate what state the unit is currently in, be it normal operational mode or bootstrap mode, and also indicates the current network connection status. A blue LED denotes power to the unit. The audio output can be monitored on the front panel ¼” (6.35mm) stereo jack socket in combination with a headphone volume knob. The analogue audio outputs can be switched to be either a fixed level output or to be controlled by the front panel volume knob. The volume can also be adjusted, as well as many other features, using an infra-red remote control (included).

The PS-PLAY can be configured with up to 3 sources. The sources are prioritized in number order. If one has failed, the next one will attempt to play. If all fail, an external USB drive will be used as a back-up source. The external USB plugs into the USB socket on the front of the unit. This enables the PS-PLAY to act as a USB audio player, playing any of the audio formats previously mentioned.  The rear panel has 2 x RJ45 connectors, one for the 10/100Mbit Ethernet interface and one for GPIO connections. The PS-PLAY has 2 output relay contacts which can triggered remotely over IP from a connected PS-SEND unit. There is a 9-way D-type RS232 serial connection for control of the unit by automation systems and for firmware updates. The unit can be remote controlled via serial connection, TCP or UDP and remote management of the unit is also possible using SNMP traps. Power to the unit is via a universal supply 85V – 264V fused IEC mains socket.

Typical Applications

- As a backup STL (studio to transmitter link).

- For audio confidence monitoring in remote locations, such as at a radio transmitter site.

- For distribution of audio and music around a building, such as for passing audio to speakers in a conference room.

- As an internet-based IP music distribution system.

- As a tannoy, paging or IP based public address system.

- For in-house audio applications and distribution.

- For streaming internet radio in bars and clubs.

- For radio and music channels in hotel rooms distributed via an IP network.

- For playing audio from a PC, a jukebox application, or from a USB stick.

Product Features

- IP to audio streamer.

- Built-in web server for configuration.

- Balanced & unbalanced analogue audio outputs.

- AES/EBU, S/PDIF and TOSlink digital audio outputs.

- Front panel headphone monitor with local & remote volume control.

- USB port acts as an audio source.

- Can decode from a multitude of services, e.g. Shoutcast, Icecast.

- 2 x GPO relays controlled remotely from a PS-SEND.

TECHNICAL SPECIFICATIONS

Technical Specification

- Analogue Outputs: 2 × XLR 3-pin (balanced, L & R)

- Analogue Outputs: 2 × RCA phono (unbalanced, L & R)

- Analogue Outputs: 1 × ¼" (6.35mm) stereo headphone jack

- Maximum Analogue Output Level: +18dBu (XLR balanced), +8dBu (RCA phono unbalanced)

- Balanced Output Impedance: <50Ω

- Unbalanced Output Impedance: <75Ω

- Analogue Output SNR: 94dB

- Frequency Response: 20Hz–20kHz (using MP3 at a bit rate of 192kbps and above)

- Output THD: 0.03% relative

- Interchannel Isolation (Crosstalk): 80dB (Ref FSD)

- Analogue Output Gain Range: -60dB to +18dB via front panel control knob or optional IR controller

- Digital Outputs: 1 × AES/EBU XLR 3-pin female

- Digital Outputs: 1 × S/PDIF RCA phono

- Digital Output: 1 × TOSLink optical

- Headphone Output: Drives 150mW into 32Ω to 600Ω stereo headphones

- USB Port: 1 × USB Type-A

- GPOs (General Purpose Outputs): 2 × switchable relay contacts (simultaneously switched), controlled from PS-SEND

- Serial Port: 1 × 9-pin D-type socket for control commands and firmware updates

- Ethernet Port: 1 × RJ45 socket supporting remote control via TCP/UDP and firmware updates

- IR Remote Receiver: Built-in IR sensor for optional remote control

- Mains Input: Filtered IEC, 85–264VAC, 47–63Hz, 10W max

- Fuse Rating: 1A anti-surge, 20 × 5mm

Audio Codec Specifications

- G.711: μ-Law/A-Law, 8kHz–48kHz sampling rate

- WAV: IMA ADPCM+ and 16-bit PCM uncompressed, 8kHz–48kHz

- MP3 MPEG-1 Layer III: 32kHz, 44.1kHz and 48kHz (CBR, VBR & ABR)

- MP3 MPEG-2 Layer III: 16kHz, 22.05kHz and 24kHz (CBR, VBR & ABR)

- Ogg Vorbis: Floor 1

Supported Network Transport Protocols

- RTP: UDP

- HTTP: TCP

- Raw UDP

- Raw TCP

- SNMP: Trap support for remote management

- Dynamic IP Address Resolution: DHCP, BOOTP, IPZator or AUTOIP

- SonicIP: IP address readout

Physical Specification

- Dimensions (Raw): 22cm (W) × 13.7cm (D) × 4.3cm (H) / 8.67" (W) × 5.39" (D) × 1.7" (H)

- Dimensions (Boxed): 34cm (W) × 27cm (D) × 6cm (H) / 13.4" (W) × 10.6" (D) × 2.4" (H)

- Weight: Nett: 0.84kg (1.85lbs), Gross: 1.64kg (3.6lbs)

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/ps-play/