RB-DDA6W

6 Way Word Clock Distribution Amplifier

The RB-DDA6W word clock distribution amplifier distributes a word clock BNC input signal to 6-word clock BNC outputs re-conditioned.

It has a single female BNC audio input which is distributed to 6 female BNC outputs. The unit’s primary application is to distribute a master TTL word clock source to multiple pieces of equipment that need to be synchronised from the master.

Typical Applications

Distribution of word clock signal from a master generator to other studio equipment to keep them synchronised.

Product Features

- 1x TTL Word clock (BNC) input.

- 6x TTL Word clock (BNC) digital audio outputs.

- Fully buffered outputs.

TECHNICAL SPECIFICATIONS

Audio Specification

- Input impedance: 110Ω ±20% balanced

- Output impedance: 110Ω ±20% balanced

- Sample Freq Range: 30–100kHz (including 32kHz, 44.1kHz, 48kHz, 64kHz, 88.2kHz and 96kHz)

- Signal Level: 2V/7V peak-to-peak (min/max)

Connections

- Audio Inputs: 1 x AES/EBU XLR 3-pin female (balanced)

- Audio Outputs: 6 x AES/EBU XLR 3-pin male (balanced)

- Alarm Outputs: 2 x Relay output indicators on 1 x 9-pin D-Type socket

- Mains Input: 2 x Filtered IEC, 85–264V switchable, fused, 6W max

- Fuse Rating: 2 x Anti-surge fuse 1A, 20 x 5mm

Physical Specification

- Dimensions (Raw): 48cm (W) x 10.8cm (D) x 4.2cm (H) (1U)

- Dimensions (Raw): 19” (W) x 4.3” (D) x 1.7” (H) (1U)

- Dimensions (Boxed): 58.5cm (W) x 22.5cm (D) x 7cm (H)

- Dimensions (Boxed): 23” (W) x 8.9” (D) x 2.8” (H)

- Weight: Nett: 1.5kg Gross: 2.0kg

- Weight: Nett: 3.3lbs Gross: 4.4lbs

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/rb-dda6w/