RB-HD6

6 Way Headphone Distribution Amplifier

The RB-HD6 headphone distribution amplifier is a 1U rack-mount which distributes stereo audio to up to 6 different sets of headphones, or can be used as 6 independent headphone amplifiers, each with their own input and volume control. A typical application might be to provide common headphone feeds for guests in a radio studio, with a separately derived feed, perhaps including talk-back, for the presenter. The main stereo input is on XLR-3 connectors on the rear panel which are electronically balanced and can be wired unbalanced. A stereo/mono input select switch on the rear panel sums left and right outputs to provide a mono feed to the headphones. The unit can receive an override audio signal via a jack socket for each output channel. Plugging in the jack plug will divert the headphone output from the master audio signal to the audio present on the jack plug. The override audio inputs can also be individually configured as parallel outputs of the front sockets, by setting internal jumpers. The master volume control adjusts overall level of the 6 outputs and does not affect the level of channels using the override inputs. The master volume can be disabled by internal jumpers.

Typical Applications

Provides multiple guest headphone outputs in a radio studio.

Product Features

- Stereo analogue balanced XLR inputs.

- 6 front panel headphone outputs.

- Individual level controls for each output.

- Master level control.

- Stereo / mono feed selection.

TECHNICAL SPECIFICATIONS

Audio Specification

- Input Impedance: >20kΩ balanced bridging, >10kΩ unbalanced override

- Output Level: Drives 150mW into 32Ω to 600Ω headphones

- Individual Volume Control: -60dB to +18dB gain

- Maximum Input Level: +28dBu

- Override Inputs: +3dBu for full volume at +18dB gain

- Master Volume Control: ±10dB gain

Connections

- Main Stereo Inputs: 2 x XLR 3-pin female (balanced, can be unbalanced)

- Insert Inputs/Parallel Outputs: 6 x ¼” (6.35mm) A-gauge 3-pole stereo jack sockets (unbalanced)

- Outputs: 6 x ¼” (6.35mm) A-gauge 3-pole stereo jack sockets

- Gains Input: Filtered IEC, switchable 110–120V or 220–240V, fused, 9W max

- Fuse Rating: Anti-surge fuse 100mA, 20 x 5mm (230VAC)

- Fuse Rating: Anti-surge fuse 250mA, 20 x 5mm (115VAC)

Physical Specification

- Dimensions (Raw): 48cm (W) x 10.8cm (D) x 4.2cm (H) (1U)

- Dimensions (Raw): 19” (W) x 4.3” (D) x 1.7” (H) (1U)

- Dimensions (Boxed): 58.5cm (W) x 22.5cm (D) x 7cm (H)

- Dimensions (Boxed): 23” (W) x 8.9” (D) x 2.8” (H)

- Weight: Nett: 1.35kg Gross: 2.0kg

- Weight: Nett: 3.0lbs Gross: 4.4lbs

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/rb-hd6/