RB-AES4X3

Quad 3 Way AES/EBU Splitter XLR Connectors

The RB-AES4X3 is a passive, quad “one-to-three” splitter housed in a 19” rack.

The RB-AES4X3 is a passive, quad “one-to-three” splitter housed in a 19” rack. Each bank is designed to split a single AES3 digital audio source to up to three destinations, using Neutrik XLR connectors.

The RB-AES4X3 provides solutions to a range of digital signal distribution problems, where correct termination is essential to maintain signal integrity. The input signal is split through high quality transformers, and 110Ω termination can be applied, if desired, to unconnected outputs.

The RB-AES4X3 requires no power to operate, ensuring your audio remains connected from source to destination(s) without interruption from power failures.

Typical Applications

Distribution of AES3 source to 3 mixing consoles.

Product Features

- 4x 3-way AES3 (XLR) splitter.

- Switched 110Ω termination on all outputs.

- Passive unit - no power supply.

TECHNICAL SPECIFICATIONS

Cable Drive Capability

- The table below sets out the minimum signal amplitude required to drive 100m (cumulative) of 110Ω twisted pair cable, based on the sample rate of the digital audio:

- 32kHz: Minimum Signal Amplitude *2Vpk-pk

- 44.1kHz: Minimum Signal Amplitude *2Vpk-pk

- 48kHz: Minimum Signal Amplitude *2Vpk-pk

- 88.2kHz: Minimum Signal Amplitude 5Vpk-pk

- 96kHz: Minimum Signal Amplitude 5Vpk-pk

- *Minimum of 2Vpk-pk is defined by the AES3 format specification.

- The table below sets out the minimum signal amplitude required to drive 30m (cumulative) of 110Ω twisted pair cable, based on the sample rate of the digital audio:

- 176.4kHz: Minimum Signal Amplitude 3Vpk-pk

- 192kHz: Minimum Signal Amplitude 3Vpk-pk

Audio Specification

- Input & Output Impedances: 110Ω

- Insertion Loss: 0.17dB 1:1, 1.3dB 1:3

- Impedance Match: +/- 2%
• Max Signal 10 V P-P: Up to 96kHz

- Attenuation Loss: 0.17dB 1:1, 6.2dB 1:3

Connections

- Inputs: 4 x XLR 3 pin female

- Outputs: 12 x XLR 3 pin male

Physical Specification

- Dimensions (Raw): 48cm (W) x 10.8cm (D) x 4.2cm (H)(1U)

- Dimensions (Raw): 19” (W) x 4.3” (D) x 1.7” (H) (1U)

- Dimensions (Boxed): 58.5cm (W) x 22.5cm (D) x 7cm (H)

- Dimensions (Boxed): 23” (W) x 8.9” (D) x 2.8” (H)

- Weight: Nett: 1.0kg Gross: 1.4kg

- Weight: Nett: 2.25lbs Gross: 3.0lbs

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/rb-aes4x3/