RB-UL4

Quad Stereo Unbalanced to Balanced Converter

The RB-UL4 is a 1U rack-mount quad stereo unit for interfacing domestic or semi-professional unbalanced equipment to professional balanced line levels.

All connections are on the rear panel. The eight RCA unbalanced inputs have an impedance of 10kΩ and are routed to eight balanced XLR-3 outputs with an output impedance of <50Ω.The output gain can be individually adjusted for left and right channels by using pre-set potentiometers accessible through the rear panel.

Typical Applications

Interfacing domestic unbalanced audio devices to balanced audio equipment.

Product Features

- Pre-set left and right gain controls

- 4x stereo phono (10kΩ) inputs.

- 4x stereo balanced Neutrik XLR <50Ω outputs.

TECHNICAL SPECIFICATIONS

Audio Specification

- Maximum Input Level: +28dBu

- Maximum Output Level: +28dBu

- Input Impedance: >10kΩ

- Output Impedance: <50Ω

- Distortion: 0.01% THD @ 1kHz, ref. +8dBu output

- Noise: -100dB, unity gain, ref. +8dBu output

- Frequency Response: 20Hz to 20kHz ±0.1dB (600Ω load, ref. 1kHz)

- Gain Range: Balanced output: -15dBu to +15dBu, ref. -15dBu into unbalanced RCA input

Connections

- Inputs: 8 x RCA phono (unbalanced)

- Outputs: 8 x XLR 3-pin male (balanced)

- Mains Input: Filtered IEC, switchable 110–120V or 220–240V, fused, 6W max

- Fuse Rating: Anti-surge fuse 100mA, 20 x 5mm (230VAC)

- Fuse Rating: Anti-surge fuse 250mA, 20 x 5mm (115VAC)

Physical Specification

- Dimensions (Raw): 48cm (W) x 10.8cm (D) x 4.2cm (H) (1U)

- Dimensions (Raw): 19” (W) x 4.3” (D) x 1.7” (H) (1U)

- Dimensions (Boxed): 58.5cm (W) x 22.5cm (D) x 7cm (H)

- Dimensions (Boxed): 23” (W) x 8.9” (D) x 2.8” (H)

- Weight: Nett: 1.3kg Gross: 1.9kg

- Weight: Nett: 2.9lbs Gross: 4.2lbs

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/rb-ul4/