RB-MS4X3

Quad 3 Way Passive Microphone Splitter

The RB-MS4X3 is a passive, quad, “one-to-three” splitter housed in a 19” rack. Each bank is designed to split a single microphone or line source to up to three destinations, using Neutrik XLR connectors. Cable connections are located on the rear panel, with recessed controls and indicators available to the user on the front panel, allowing quick and easy access to setup parameters.

Wide Signal Range Capability

The RB-MS4X3 uses high quality audio transformers that are capable of accepting input levels of up to +18dBu, making the splitter useful in both microphone and line level splitting applications. A 30dB pad can be applied to the input, allowing a line level signal to be interfaced into a microphone input with suitable levels and the correct termination.

Versatile Phantom Powering Options

The RB-MS4X3 offers three methods of providing phantom power to a microphone connected to its input:

- From the external +48VDC power connector.

- From a microphone amplifier connected to output 1 (direct).

- From a microphone amplifier connected to output 2 (phantom loopback).

Controlling which method is used to provide phantom power is easy with front panel switches and indicators. It is possible to concurrently power a microphone using any two of the above methods without degrading audio performance, thus providing power supply redundancy.

Ground Loop Hum Elimination

Ground loop hum problems can be quickly eliminated using the front panel toggle switches to lift pin 1 of the output connectors (output 2 and 3 only).

Typical Applications

To share microphones between radio/TV/production studios.

Product Features

- 4x 3-way mic splitter.

- Balanced XLRs.

- Passive transformer design.

- Ground lift switches.

- External or looped phantom power inputs.

- Line level -30dB pad.

TECHNICAL SPECIFICATIONS

Test

- Frequency Response: 10Hz to 30kHz ±0.5dB
Ref. -6dBu, 1kHz ±0.5dB
Source impedance = 150Ω
Load impedance = 10kΩ

- Total Harmonic Distortion: 0.02%
Ref. +3dBu, 50Hz
Source impedance = 150Ω
Load impedance = 10kΩ

- Total Harmonic Distortion (0.1%): +13dBu
Ref. 0.1% THD+N, 50Hz
Source impedance = 150Ω
Load impedance = 10kΩ

- Total Harmonic Distortion (1%): +18dBu
Ref. 1% THD+N, 50Hz
Source impedance = 150Ω
Load impedance = 10kΩ

- Common Mode Rejection Ratio: >60dB
Ref. 20kHz
Source impedance = 600Ω
Load impedance = 10kΩ

Connections

- Inputs: 4 x XLR 3-pin female

- Outputs: 12 x XLR 3-pin male

- DC Phantom Power Input: +48V DC, 2.5mm socket

Physical Specification

- Dimensions (Raw): 48cm (W) x 10.8cm (D) x 4.2cm (H) (1U)

- Dimensions (Raw): 19” (W) x 4.3” (D) x 1.7” (H) (1U)

- Dimensions (Boxed): 58.5cm (W) x 22.5cm (D) x 7cm (H)

- Dimensions (Boxed): 23” (W) x 8.9” (D) x 2.8” (H)

- Weight: Nett: 1.5kg Gross: 1.9kg

- Weight: Nett: 3.3lbs Gross: 4.2lbs

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/rb-ms4x3/