RB-DA6G

6 Way Stereo Distribution Amplifier with Output Gain Control

The RB-DA6G is a 1U rack-mount high performance 6 way stereo distribution amplifier for splitting a source into a number of different outputs.

The RB-DA6G is a 1U rack-mount high-performance 6 way stereo distribution amplifier for splitting a source into a number of different outputs. The RB-DA6G has one stereo input and six stereo outputs. It can also be configured to distribute a single mono input to twelve outputs. This is enabled via a recessed front-panel switch to prevent unintended switching. The XLR-3 inputs and outputs are electronically balanced and can be wired unbalanced. Each output is individually buffered so that a short circuit on one output won’t affect the others. Individual output gain may be varied from -8dB to 18dB, which is useful for normalising consumer and professional signals to give outputs of -15dBu and 0dBu respectively.

Typical Applications

Distribute line level signals to multiple reporters in a broadcast centre, distribute main programme output from a mixer to other studios, create multiple feeds of a recorded signal at different output levels.

Product Features

- 2x balanced analogue inputs.

- 12x balanced analogue outputs.

- 6x stereo / 12 x mono distribution modes.

- -8dB to 18dB pre-set gain controls for every output.

TECHNICAL SPECIFICATIONS

Audio Specification

- Maximum Input Level: +28dBu

- Maximum Output Level: +28dBu

- Frequency Response: 20Hz to 20kHz ±0.1dB (600Ω load, ref 1kHz)

- Gain Range: Adjustable 8dB loss to 18dB gain (12 adjustable pots)

- Common Mode Rejection: >66dB typically

- Input impedance: >20kΩ bridging

- Output impedance: <50Ω

- Distortion: 0.01% THD @ 1kHz, ref +8dBu output

- Noise: -88dB unity gain, ref +8dBu output

Connections

- Inputs: 2 x XLR 3 pin female (balanced, can be unbalanced)

- Outputs: 12 x XLR 3 pin male (balanced, can be unbalanced)

- Gains Input: Filtered IEC, switchable 110-120V, or 220-240V, fused, 6W max

- Fuse Rating: Anti-surge fuse 100mA 20 x 5mm (230VAC)

- Fuse Rating: Anti-surge fuse 250mA 20 x 5mm (115VAC)

Physical Specification

- Dimensions (Raw): 48cm (W) x 10.8cm (D) x 4.2cm (H) (1U)

- Dimensions (Raw): 19” (W) x 4.3” (D) x 1.7” (H) (1U)

- Dimensions (Boxed): 58.5cm (W) x 22.5cm (D) x 7cm (H)

- Dimensions (Boxed): 23” (W) x 8.9” (D) x 2.8” (H)

- Weight: Nett: 1.3kg Gross: 1.9kg

- Weight: Nett: 2.9lbs Gross: 4.2lbs

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/rb-da6g/