RB-DAC1

Digital to Analogue Converter

Using 24 bit, 192kHz capable devices, the RB-DAC1 D/A Converter is a 1U rack-mount which produces a stereo balanced XLR or unbalanced phono output from an incoming AES/EBU or S/PDIF digital input signal. There is also a headphone output with volume control for monitoring purposes.

The analogue outputs have an output level control, allowing full-scale settings selectable from +12dBu, +18dBu or +24dBu. The RCA phono outputs have a further 8.5dBu attenuation incorporated.

There is a button to select either the AES/EBU or S/PDIF input for the D/A converter, which is located on the rear panel. Additionally, if de-emphasis is selected, the RB-DAC1 will decode 50/15µs emphasis when indicated by certain channel status bits in the incoming digital audio data.

When operating, the front panel power LED flashes whenever the unit is not synchronised to the incoming digital signal.

Typical Applications

To connect the output of a CD player or digital playout system to an analogue amplifier, or the program output of a digital mixer to a pair of powered speakers.

Product Features

- AES/EBU & S/PDIF digital inputs.

- Balanced XLR and phono analogue outputs.

- Headphones output with level control.

- Switchable de-emphasis decode.

TECHNICAL SPECIFICATIONS

Audio Specification

- Maximum Output Level: Balanced Output: +24dBu

- Maximum Output Level: Unbalanced Output: +15.5dBu

- Maximum Output Level: Headphone Output: +20dBu (ref. <0.01% THD)

- Output Impedance: Balanced Output: <50Ω

- Output Impedance: Unbalanced Output: <75Ω

- Output Impedance: Headphone Output: Drives 150mW into 32Ω to 600Ω professional headphones

- Dynamic Range: Balanced Output: 115dB

- Dynamic Range: Unbalanced Output: 115dB

- Dynamic Range: Headphone Output: 100dB

- Gain Range: Balanced Output: Selectable 12dBu, 18dBu or 24dBu output level, ref FSD

- Gain Range: Unbalanced Output: Selectable 3.5dBu, 9.5dBu or 15.5dBu output level, ref FSD

- Gain Range: Headphone Output: -80dBu (volume 0%) to +20dBu (volume 100%) ref FSD

- Distortion & Noise: Balanced Output: <0.005% THD+N @1kHz, ref +8dBu

- Distortion & Noise: Unbalanced Output: <0.005% THD+N @1kHz, ref -6dBV

- Distortion & Noise: Headphone Output: <0.005% THD+N @1kHz, ref +8dBu

- Sample Freq. Range: Balanced Output: 32kHz - 192kHz

Operational Controls

- Digital Input Select: AES/EBU or S/PDIF, via push-button

- Gain Select: DIP switch

- De-emphasis On/Off: DIP switch

Connections

- Digital Inputs: 1 x AES/EBU XLR 3 pin female

- Digital Inputs: 1 x S/PDIF RCA phono

- Analogue Outputs: 2 x XLR 3 pin male (balanced)

- Analogue Outputs: 2 x RCA phono (unbalanced)

- Headphone Output: 1 x ¼” (6.35mm) A/B-gauge 3-pole stereo jack sockets

- Mains Input: Filtered IEC, 110V-120V, or 220-240V, fused, 18W max

- Fuse Rating: Anti-surge fuse 100mA 20 x 5mm (230VAC), Anti-surge fuse 250mA 20 x 5mm (115VAC)

Physical Specification

- Dimensions (Raw): 48cm (W) x 10.8cm (D) x 4.2cm (H) (1U)

- Dimensions (Raw): 19” (W) x 4.3” (D) x 1.7” (H) (1U)

- Dimensions (Boxed): 58.5cm (W) x 22.5cm (D) x 7cm (H)

- Dimensions (Boxed): 23” (W) x 8.9” (D) x 2.8” (H)

- Weight: Nett: 1.4kg Gross: 2.0kg

- Weight: Nett: 3.1lbs Gross: 4.4lbs

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/rb-dac1/