RM-MC4L

Monitor Controller, 4 Stereo Inputs, 1 Stereo Output with Light Control

The RM-MC4L has four stereo balanced analogue inputs with a single stereo balanced output, all using XLR connectors.

The RM-MC4L has four stereo balanced analogue inputs with a single stereo balanced output, all using XLR connectors. Four illuminated front panel buttons allow you to simply select the source required at the output.

The RM-MC4L has the same master volume control, CUT, DIM and light control functions and adjustments as the RM-MC1L unit.

Typical Applications

Production control room, live studio broadcasts, recording studios.

Product Features

- 4x Stereo balanced analogue inputs (XLR).

- 1x Stereo balanced analogue output (XLR).

- 4x source selector buttons.

- 0dB to -86dB master volume control.

- Separate remoteable DIM & CUT buttons.

- Tally outputs for REH (Rehearse), Standby, and TX light control.

- 2 minute Standby countdown timer feature.

- DIM level control (rear panel pre-set pot).

TECHNICAL SPECIFICATIONS

Inputs

- Input Impedance: >20kΩ

- Maximum Input Level: +25dBu

- Output Impedance: <50Ω

- Maximum Output Level: +25dBu

- Attenuation Range: >86dB

- DIM Attenuation Range: Adjustable from -3dB to -23dB via rear panel potentiometer

- Common Mode Rejection: >66dB typical

- Distortion: <0.04% (referenced to +8dBu)

- Crosstalk: <-80dB @ 1kHz input, referenced to 0dBu

Front Panel Operation Controls

- Light Control (All Units): Transmit (TX), Standby and Rehearse (REH) illuminated buttons

- Channel Selection: 1, 2, 3 & 4 illuminated buttons

- Audio Controls: DIM and CUT illuminated buttons

- Volume Control: Front panel rotary potentiometer for output attenuation

Rear Panel Connections

- Audio Inputs: 8 × XLR 3-pin female (balanced)

- Audio Outputs: 2 × XLR 3-pin male (balanced)

- Remote I/O Port: 15-pin D-type socket

- Mains Input: Filtered 3-pin IEC male, 85–264VAC, 47–63Hz, fused

- Fuse Rating: 1A anti-surge, 20 × 5mm

Physical Specification

- Dimensions (Raw): 48cm (W) × 10.8cm (D) × 4.3cm (H) / 19" (W) × 4.3" (D) × 1.7" (H) (1U)

- Dimensions (Boxed): 58.5cm (W) × 22.5cm (D) × 7cm (H) / 23" (W) × 8.9" (D) × 2.8" (H)

- Weight: Nett: 1.3kg (2.9lbs), Gross: 1.9kg (4.2lbs)

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/rm-mc4l/