RM-MC1L

Monitor Controller, Single Stereo Input & Output with Light Control

The RM-MC1L is a stereo unit with a single stereo balanced analogue input and output on XLRs.

The RM-MC1L is a stereo unit with a single stereo balanced analogue input and output on XLRs. Useful in a production environment, the unit allows you to control the audio level to external monitors and to DIM the audio, or CUT it completely with simple front panel controls, or remote external inputs. It has a master volume control that adjusts the attenuation on the output channels from 0dB down to -86dB. A CUT control mutes the audio outputs when enabled, whilst a DIM control attenuates the audio output channels by a pre-set amount from -2dB to -23dB. All of the buttons on the front panel are illuminated by LEDs and the brightness can be changed to better suit the environment in which the unit is used.

Typical Applications

Production control room, live studio broadcasts, recording studios.

Product Features

- Stereo balanced analogue input and output (XLR).

- 0dB to -86dB master volume control.

- Separate remoteable DIM & CUT buttons.

- Tally outputs for REH (Rehearse), Standby, and TX light control.

- 2-minute Standby countdown timer feature.

- DIM level control (rear panel pre-set pot).

TECHNICAL SPECIFICATIONS

Inputs

- Input Impedance: >20kΩ

- Maximum Input Level: +25dBu

- Output Impedance: <50Ω

- Maximum Output Level: +25dBu

- Attenuation Range: >86dB

- DIM Attenuation Range: Adjustable from -3dB to -23dB via rear panel potentiometer

- Common Mode Rejection: >66dB typical

- Distortion: <0.04% (referenced to +8dBu)

- Crosstalk: <-80dB @ 1kHz input, referenced to 0dBu

Front Panel Operation Controls

- Light Control (All Units): Transmit (TX), Standby and Rehearse (REH) illuminated buttons

- Audio Controls: DIM and CUT illuminated buttons

- Volume Control: Front panel rotary potentiometer for output attenuation

Rear Panel Connections

- Audio Inputs: 2 × XLR 3-pin female (balanced)

- Audio Outputs: 2 × XLR 3-pin male (balanced)

- Remote I/O Port: 15-pin D-type socket

- Mains Input: Filtered 3-pin IEC male, 85–264VAC, 47–63Hz, fused

- Fuse Rating: 1A anti-surge, 20 × 5mm

Physical Specification

- Dimensions (Raw): 48cm (W) × 10.8cm (D) × 4.3cm (H) / 19" (W) × 4.3" (D) × 1.7" (H) (1U)

- Dimensions (Boxed): 58.5cm (W) × 22.5cm (D) × 7cm (H) / 23" (W) × 8.9" (D) × 2.8" (H)

- Weight: Nett: 1.3kg (2.9lbs), Gross: 1.9kg (4.2lbs)

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/rm-mc1l/