RM-4C8

Reference Monitor, 4 LED Meters, 8 Channel Inputs & Dual Source Selectors

The RM-4C8 is ideal for monitoring audio channels in an SDI group, or groups of de-embedded AES/EBU channels.

With 4 x bright high-resolution 26 segment meter displays and separate left and right source selectors, the RM-4C8 is ideal for monitoring audio channels in an SDI group, or groups of de-embedded AES/EBU channels. The RM-4C8 offers the same functionality as the RM-2S4 but with an additional source selector so that any of the 4 channels in the selected group, or bank, can be monitored independently on left and right speakers. Also, 4 meters are provided so that every channel in the selected group, or bank, can be visually monitored and 4 expansion port groups of 4 channels are allowed, so that all audio channels in an SDI signal can be monitored, using the optional RM-HD1 card.

The audio inputs on the RM-4C8 are auto-sensing, for digital AES/EBU using the left input XLR, or both XLRs for analogue inputs. The inputs can be used in any combination of analogue or digital. With full remote control via GPI, RS232 or USB, a 5 band parametric equaliser, 6 front panel modifier buttons and the ability to take the optional 3G/HD/SD-SDI expansion cards, the RM-4C8 is a flexible and versatile monitoring solution. The same high level of care has been taken in the design of the RM-4C8 as in the RM-2S4 to ensure that it will be the best sounding 1U rack-mount audio monitor that you listen to. This product has settings that can be adjusted using the Sonifex SCi (Serial Control Interface) software.

Typical Applications

Monitoring of multiple audio sources in an OB truck or engineering racks room.

Product Features

- 2 x banks of four stereo analogue/digital auto-sensing inputs as standard.

- Optional input expansion cards available.

- Option for HD/SD-SDI de-embedding.

- Stereo analogue / digital output follows source selection.

- Independent L/R output source selection.

- Selected 4-channel source metering with selectable scales / responses.

- Phase meters for correlation / phase error indication.

- Audio status alarm outputs.

- Bank of push-button audio modifiers.

- 5 band parametric EQ

- 3-way speakers with Class D amplification and DSP crossover.

- Headphones output.

- RS232 / USB control and configuration - Sonifex SCi software.

TECHNICAL SPECIFICATIONS

Inputs

- Audio Inputs: 8 analogue or AES/EBU digital channels (auto-selecting)

- Maximum Input Level (0dB gain): +18dBu (analogue) / 0dBFS (digital)

- CMRR: >60dB typical

- Input Impedance: 20kΩ (analogue), 110Ω (digital with switchable termination)

- AES/EBU Sample Rate: 32kHz–192kHz, internally converted to 48kHz

- Input Gain: 0, +6, +12 or +18dB digital gain (switchable)

- Input Selection: 2 × front panel rotary controls with indicator LEDs

Line Level Outputs

- Audio Outputs: 1 × stereo analogue or AES/EBU digital (switchable)

- Gain Relative to Selected Input: Unity or variable, following volume control (switchable)

- Maximum Output Level: +18dBu (analogue) / 0dBFS (digital)

- Output Impedance: <50Ω (analogue), 110Ω (digital)

- AES/EBU Sample Rate: 48kHz

- Distortion: <0.02% (1kHz, +8dBu output)

- Noise: -84dB RMS, unity gain referenced to +8dBu output

- Frequency Response: 20Hz–20kHz, +0/-0.5dB

- Crosstalk (Analogue I/O):

- 1kHz input: <-90dB

- 10kHz input: <-85dB

Audio Modifiers

- Modifier Selection: Illuminated front panel pushbuttons

- DIM: Reduces speaker audio level by 10dB

- CUT L / CUT R: Mutes left or right speaker audio

- MONO: Combines left and right audio inputs

- PHASE INVERT: Inverts phase of the right audio input

- M+S: Converts stereo input to Mid (sum) and Side (difference) signals

User-Variable Equalisation

- Type: Parametric

- Bands: Five

- Centre Frequency: 200Hz–18kHz

- Bandwidth: 0.25 to 2 octaves

- Boost/Cut: ±12dB

- Programming: Via USB/serial control port

Amplifier & Loudspeakers

- Configuration: Three-way with stereo mid/high-frequency drivers and mono low-frequency driver

- Power Output: 2 × 5W (HF) + 20W (LF) with protective limiter

- Crossover: 500Hz (3rd-order Butterworth)

- HF Distortion: <0.05% (1kHz, 3W output)

- LF Distortion: <0.05% (100Hz, 6W output)

- Noise: More than 80dB below full output

- Volume Control: Mute to full volume via front panel rotary control

- Balance Trim: ±6dB via front panel rotary control

- Peak Acoustic Level: 102dB SPL @ 2ft

Level Metering

- Meters: 4 × 26-segment tri-colour LED bargraphs

- Meter Characteristics:

- Dual BBC PPM + Standard VU

- BBC PPM

- EBU PPM

- Nordic PPM

- AES/EBU Digital PPM

- DIN PPM

- Standard VU

- Extended VU

- Ballistics: According to selected characteristic

- Line-Up Level: According to selected characteristic

Phase Metering

- Type: 5-segment display indicating 0°, 45°, 90°, 135° and 180°

Remote Control

- USB: Slave device, 19200 baud

- Serial: RS232, 19200 baud, 3-wire connection

- Alarm Outputs:

- Audio under-level/fail (latching)

- Audio over-level (latching)

- Sustained phase error (latching)

- AES/EBU input unlock (non-latching)

- Open-collector outputs rated at 30V, 50mA maximum

- Output low/conducting in normal (no alarm) condition

- Control Inputs:

- Mute audio

- Dim audio

- Alarm reset

- Pull-to-ground activation

Status Indicators

- LIMIT: Indicates loudspeaker protection limiter is active

- CLIP: Indicates internal digital clipping due to over-level

- LOCK: Indicates lock achieved on selected digital input(s)

- OPT: Reserved for future use

Connectors

- Audio Inputs: 8 × XLR 3-pin female (balanced, may be unbalanced)

- Audio Outputs: 2 × XLR 3-pin male (balanced, may be unbalanced)

- Headphones: ¼" (6.35mm) A-gauge 3-pole stereo jack socket

- USB: Type-B socket

- Serial: 9-pin D-sub female

- Remote I/O: 15-pin D-sub male

- Mains Input: Filtered 3-pin IEC male, 85–264VAC, 47–63Hz, fused, 60W peak, 30W average

- Fuse Rating: 2A anti-surge, 20 × 5mm

Physical Specification

- Dimensions (Raw): 48cm (W) × 30.5cm (D) × 4.4cm (H) (1U) / 19" (W) × 12" (D) × 1.73" (H)

- Dimensions (Boxed): 57cm (W) × 52cm (D) × 15cm (H) / 22.4" (W) × 20.5" (D) × 5.9" (H)

- Weight: Nett: 4.5kg (10.0lb), Gross: 6.0kg (13.2lb)

Options

- RM-HD1: 3G/HD/SD-SDI expansion card

- RM-HDE1: 3G/HD/SD-SDI & Dolby® E expansion card

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/rm-4c8/