AVN-AIO4

4 Input, 4 Output Dante® Interface, PoE

The AVN-AIO4 converts up to 4 channels of analogue in and 4 channels of analogue out to and from the Dante network.

The AVN-AIO4 audio converter and interface converts up to four analogue inputs and four analogue outputs to and from the Dante Audio-over-IP networking standard. This cost effective 1U rack-mount unit offers an easy solution for AV professionals and system integrators. It is simple to configure and operate, with all set-up, except line-up levels, done via the standard Dante Controller software and power via PoE (Power Over Ethernet). All analogue inputs and outputs are on high-quality Neutrik XLR connectors and there are front panel status/confidence LEDs for PoE, Link, and Clock. Global 0dBFS line-up can be set to +12dBu, +18dBu or +24dBu to meet your specific requirement via the front panel recessed toggle switch.

Product Features

- 4 x balanced analogue inputs on XLR.

- 4 x balanced analogue outputs on XLR.

- Adjustable global 0dBFS line up selection (+12dBu/+18dBu/+24dBu).

- 1 x RJ45 Dante connector (1Gb/s Ethernet Port).

- PoE, Link, and Clock LED status indicators.

- Configuration using Dante Controller.

- AES67 operation & Dante Domain Manager compliant.

- Powered by PoE.

- 1U 19” rack-mount form factor.

Accessories

- AVN-AIORK - 1U Rear Rack Kit For AVN-AIO/M Range.

Typical Applications

The AVN-AIO4 is suited to a wide range of professional audio environments where reliable, low-latency transport between analogue and networked systems is required. It is ideal for broadcast studios, live production and events, OB trucks, and fixed AV installations such as corporate, education, and hospitality spaces. It also integrates seamlessly into houses of worship and recording studios, providing flexible, high-quality connectivity between traditional analogue equipment and Dante-based audio networks.

Product Category

Analogue Dante Converters

TECHNICAL SPECIFICATIONS

Network and AoIP

- AoIP Standard: Dante

- Number of Channels: 4 receive, 4 transmit

- Number of Streams: 4 receive, 4 transmit

- Sample Rates: 44.1kHz, 48 kHz, 88.2kHz or 96kHz

(Note: 2 channels I/O only when >48kHz. Either 4 inputs or 4 outputs at 96kHz, can be configured on request with a separate firmware download).

- Format: Linear PCM, 16, 24 or 32 bit

- AES67 Support: Yes

- Connectivity: 1 x RJ45

- Speed: 100Mbps

- Dante Domain Manager Ready: Yes

Audio Input Performance Specifications

- Input Impedance: >20kΩ balanced

- 0dBFS line-up: User Selectable +12dBu/+18dBu/+24dBu balanced

- Frequency Response: 20Hz to 20kHz, +0/-0.2dB

- THD+N: <-110dBFS, -30dBFS, 20Hz to 20kHz, 20kHz BW

- Noise: -110dBFS, 20kHz BW, Rs=200Ω

- Crosstalk: <-100 dB

- Common Mode Rejection: >60dB @ 1kHz

Audio Output Performance Specifications

- Output Impedance: <50Ω balanced

- 0dBFS line-up: User Selectable +12dBu/+18dBu/+24dBu balanced

- Frequency Response: 20Hz to 20kHz, +0/-0.5dB

- THD+N:	<-107dBFS, -30dBFS, 20Hz to 20kHz, 20kHz BW

- Noise: -107dBFS, 20kHz BW

- Crosstalk: <100dB

Balanced Line Input XLR Pinout

- Chassis Ground/Screen

- Input Phase/Positive

- Input Non-Phase/Negative

Balanced Line Output XLR Pinout

- Chassis Ground/Screen

- Output Phase/Positive

- Output Non-Phase/Negative

PoE Power

- Standard: 802.3af

- Class: 0

- PD Power Range: 0.44 W to 12.94 W

- Typical PSE Power Usage: 4.5 W

- Max PSE Power Usage: 15.4 W

Physical Specification

- Dimensions (Raw): 48cm (W) x 11cm (D) x 4.3cm (H)(1U)

- Dimensions (Raw): 19” (W) x 4.3” (D) x 1.7” (H)

- Weight:	Nett: 0.9kg Gross: 1.4kg

- Weight:	Nett: 1.9lbs Gross: 3.1lbs

To view the product handbook or latest firmware, visit our website - https://sonifex.co.uk/product/avn-aio4/